Copyright 2019 AIA
MasterSpec Full Length
12/19 (PM 04/22)
PRODUCT MASTERSPEC LICENSED BY DELTEK, INC. TO ACORN ENGINEERING; MORRIS GROUP INTERNATIONAL

Copyright 2019 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA
This Product MasterSpec Section is licensed by Deltek, Inc. to Acorn Engineering; Morris Group International ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Acorn Engineering; Morris Group International, 15125 Proctor Ave., City of Industry, CA 91746; Phone: (626) 336-4561 | (800) 488-8999; Fax: <Insert Fax number>; Website: www.acorneng.com; Email: <Insert Email address>.

For information about MasterSpec, contact Deltek at (800) 424-5080 or visit masterspec.com.

SECTION 224213.16 - COMMERCIAL URINALS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com.>
Revise this Section by deleting and inserting text to meet Project-specific requirements.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Stall urinals.

2. Wall-hung urinals.

3. Waterless urinals.

4. Urinal flushometer valves.

5. Supports.

B. Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 224600 "Security Plumbing Fixtures" for security urinals.

1.2 ACTION SUBMITTALS
Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.
A. Product Data: For each type of product.

1. Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for urinals.

2. Include rated capacities, operating characteristics, electrical characteristics, and furnished specialties and accessories.
Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.
B. Sustainable Design Submittals:

"Product Data" Subparagraph below applies to LEED (including v4), IgCC, and Green Globes for fixtures and equipment that consume water.

1. Product Data: For water consumption.
Retain "Shop Drawings" Paragraph below if equipment includes wiring.

C. Shop Drawings: Include diagrams for power, signal, and control wiring.

1.3 CLOSEOUT SUBMITTALS
See Section 017700 "Closeout Procedures" for submission of maintenance material items.
A. Operation and Maintenance Data: For flushometer valves[ and electronic sensors] to include in operation and maintenance manuals.

1.4 MAINTENANCE MATERIAL SUBMITTALS

A. Furnish extra materials that are packaged with protective covering for storage and identified with labels describing contents.

1. Flushometer-Valve Repair Kits: Equal to [10] <Insert number> percent of amount of each type installed, but no fewer than [one] [six] <Insert number> of each type.

2. Waterless Urinal Trap-Seal Cartridges: Equal to [200] <Insert number> percent of amount of each type installed, but no fewer than [12] <Insert number> of each type.

3. Waterless Urinal Trap-Seal Liquid: Equal to [1 gal. (3.8 L)] <Insert volume> for each urinal installed.

PART 2 -  PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Deltek. Before inserting names, verify that products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Do not use same urinal designation as is used for another urinal in Section 224600 "Security Plumbing Fixtures."

2.1 STALL URINALS

Copy "Urinals - Stall, Washout Type" Paragraph below and re-edit for each type of stall, wash-out-type urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

A. Urinals - Stall, Washout Type: <Insert drawing designation>.
Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require products from Linx Industries or a comparable product from other manufacturers.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International; Dura-Ware Model 2180 <Insert drawing designation> or comparable product by one of the following:

a. Metcraft Industries, Inc.
b. Willoughby Industries.
c. <Insert manufacturer's name>.

Caution: This urinal type requires a drainage piping trap under the urinal. This trap is not part of the urinal fittings.

2. Urinal Fixture:

a. Standards:
Retain one or more of first three subparagraphs below when compliance with referenced standards is required to comply with Project design objectives, or when required by state or local code or AHJ.

1) ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant unit is required.

2) ICC A117.1.

Retain first subparagraph below when U.S. Energy Policy Act applies, and units are specified with a 1.0 gpf (3.8 Lpf) flush valve.

3) U.S. Energy Policy Act of 1992 for Water Closets.

b. Material: 14 gauge minimum-thickness, Type 304 stainless steel.
c. Finish: [No. 4 satin finish on exposed surfaces, and matte finish on interior wetted surfaces] [Powder coated finish on exposed surfaces] <Insert color from manufacturer's color chart>.
d. Bowl:

1) Type: Washout, with [back wall washdown] [continuous flush rim].

2) Rim Height: Install stall trough flush with finished floor, for ADA compliance.
e. Inlet Connection: NPS 3/4 (DN 19) male spud, [exposed top] [concealed wall] supply.

f. Bottom Outlet Connection: NPS 2 (DN 50) for inside caulk waste.

g. Water Consumption, Washout Fixtures: [1.0 gal. (3.8 L)] [1.5 gal. (5.7 L)] per flush.

h. Supply Pressure: 25 psig (172 kPa), minimum.

i. Drain: Beehive dome strainer, tamper-resistant fasteners.
j. Mounting: Installed from fixture side to reinforced wall with backing that matches fixture-mounting locations.
Coordinate "Flushometer Valve" Subparagraph below with "Urinal Flushometer Valves" Article.

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.
a. Flushometer Valve Type: [Mechanical] [Hydraulic] [Electronic].
b. Flushometer-Valve Accessories: [Top supply flush valve cover] [Wall supply flush valve access panel].

c. Inlet Connection: NPS 3/4 (DN 19), [top] [back] spud.
Retain "Power" Subparagraph below for electronically controlled flushometer valves.

d. Power: 120 Vac <Insert power source>.

All accessories listed in "Accessories" Subparagraph below are optional. Retain only subparagraphs needed to comply with Project design requirements.
4. Accessories:

a. Flush valve through wall connection.

"Transformer" Subparagraph below is available with basis-of-design product manufacturer's products equipped with Acorn's "Model MVC Time-trol" electronic flush valve.

b. Transformer: 120 V ac to 24 V ac.

2.2 WALL-HUNG URINALS

Copy "Urinals - Wall Hung, Back Outlet, Blowout" Paragraph below and re-edit for each type of wall-hung, back-outlet, blowout urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

A. Urinals - Wall Hung, Back Outlet, Blowout: <Insert drawing designation>.
Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require products from Linx Industries or a comparable product from other manufacturers.

When wall-hung urinals by Acorn Engineering are selected, retain model 2160 for regular housing design, and model 2165 for straddle housing design. Model 2165 complies with ADA standards when rim height is mounted to conform to ADA requirements.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International; Dura-Ware Model [2160] [2165] <Insert drawing designation> or comparable product by one of the following:

a. Metcraft Industries, Inc.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Urinal Fixture:

a. Standards:

Retain one or both of first two subparagraphs below when compliance with referenced standards is required to comply with Project design objectives, or when required by state or local code or AHJ.

1) ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant unit is required.

2) ICC A117.1.

b. Material: 14 gauge minimum-thickness, Type 304 stainless steel.

c. Finish: [No. 4 satin finish on exposed surfaces, and matte finish on interior wetted surfaces] [Powder coated finish on exposed surfaces] <Insert color from manufacturer's color chart>.
d. Bowl:

1) Type: Blowout, with [back wall washdown] [continuous flush rim].

Retain one or both of first two subparagraphs below. Only straddle type fixtures are suitable for ADA compliance.

2) Rim Height: [20 inches (508 mm), straddle configuration,] [24 inches (610 mm)] <Insert value> above finished floor.

3) Rim Height for ADA-Compliant Installation: [17 inches (432 mm)] <Insert value> above finished floor, straddle fixture configuration.

e. Inlet Connection: 1-1/2 inch NPT male, [top] [back] spud.

f. Back Outlet Connection: NPS 2 (DN 50) P-trap with O-ring adapter for 2 inch.

g. Water Consumption, Blowout Fixtures: 3.5 gal. (13.2 L) per flush.

h. Supply Pressure: 25 psig (172 kPa), minimum.

i. Mounting: Installed from fixture side to reinforced wall with backing that matches fixture-mounting locations.

Coordinate "Flushometer Valve" Subparagraph below with "Urinal Flushometer Valves" Article.

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.
a. Flushometer Valve Type: [Mechanical] [Hydraulic] [Electronic].

b. Flushometer-Valve Accessories: [Top supply flush valve cover] [Wall supply flush valve access panel].

c. Inlet Connection: NPS 3/4 (DN 19), [top] [back] spud.

Retain "Power" Subparagraph below for electronically controlled flushometer valves.

d. Power: 120 Vac.

All accessories listed in "Accessories" Subparagraph below are optional. Retain only subparagraphs needed to comply with Project design requirements. Waste adapters shown enable Installer to connect waste from fixture side of wall.

4. Accessories:

a. Flush valve through wall connection.

b. Waste adapter to 2 inch NCT, female thread.

c. Waste adapter to 2 inch NPT, male thread.

d. Waste adapter to 2 inch NPT, female thread.

e. Waste adapter to NPS 2, no-hub.
"Transformer" Subparagraph below is for applications of basis-of-design product manufacturer's products equipped with Acorn's "Model MVC Time-trol" electronic flush valve.

f. Transformer: 120 V ac to 24 V ac.

Copy "Urinals - Wall Hung, Back Outlet, Siphon Jet" Paragraph below and re-edit for each type of wall-hung, back-outlet, siphon-jet urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

B. Urinals - Wall Hung, Back Outlet, Siphon Jet: [Accessible] <Insert drawing designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Briggs Plumbing Products, Inc.

c. Duravit USA, Inc.

d. Gerber Plumbing Fixtures LLC.
e. Kohler Co.

f. Mansfield Plumbing Products LLC.

g. Zurn Industries, LLC.

h. <Insert manufacturer's name>.

2. Fixture:

a. Standards: ASME A112.19.2/CSA B45.1 and ASME A112.19.5/CSA B45.15.

b. Material: Vitreous china.

Option in "Type" Subparagraph below may not be available from some manufacturers. Extended shields vary in shape and dimensions. Verify availability and suitability if required.

c. Type: Siphon jet[ with extended shields].

d. Strainer or Trapway: [Manufacturer's standard strainer] <Insert strainer> with integral trap.

In "Water Consumption" Subparagraph below, not all manufacturers offer each option. Consult manufacturers.

e. Water Consumption: [0.125 gpf (0.5 Lpf)] [0.25 gpf (1.0 Lpf)] [0.5 gpf (1.9 Lpf)] [1.0 gpf (3.8 Lpf)].

First option in "Spud Size and Location" Subparagraph below is available from some manufacturers. Back spud is used with a remote hard-wired, solenoid-actuator, electronic-sensor flushometer valve. Verify availability if required.

f. Spud Size and Location: NPS 3/4 (DN 20); [back] [top].

g. Outlet Size and Location: NPS 2 (DN 50); back.

h. Color: [White] <Insert color>.

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.

4. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2 for coupling.

b. Size: NPS 2 (DN 50).

Coordinate "Support" Subparagraph below with "Supports" Article.

5. Support: [Type I urinal carrier] <Insert carrier> with fixture support plates and coupling with seal and fixture bolts and hardware matching fixture.[ Include rectangular, steel uprights.]

6. Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Copy "Urinals - Wall Hung, Back Outlet, Washout" Paragraph below and re-edit for each type of wall-hung, back-outlet, washout urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

C. Urinals - Wall Hung, Back Outlet, Washout: [Accessible] <Insert drawing designation>.
Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require products from Linx Industries or a comparable product from other manufacturers.

When wall-hung urinals by Acorn Engineering are selected, retain model 2158 or 2167 for straddle housing design that complies with ADA standards when rim height is mounted to conform to ADA requirements.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International; Dura-Ware Model [2158] [2162] [2167] [2173] [2174] [2175] [2176] [2178] <Insert drawing designation> or comparable product by one of the following:

a. Metcraft Industries, Inc.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Urinal Fixture:

a. Standards:

Retain one or both of first two subparagraphs below when compliance with referenced standards is required to comply with Project design objectives, or when required by state or local code or AHJ.

1) ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant unit is required.

2) ICC A117.1.

In "Material" Subparagraph below, retain product manufacturer's standard 16 gauge material thickness for all units except Model 2158, which is fabricated from 16 gauge material.

b. Material: [14] [16] gauge minimum-thickness, Type 304 stainless steel.

c. Finish: [No. 4 satin finish on exposed surfaces, and matte finish on interior wetted surfaces] [Powder coated finish on exposed surfaces] <Insert color from manufacturer's color chart>.
d. Bowl:

1) Type: Washout, with [back wall washdown] [continuous flush rim].

Retain one or both of the first two subparagraphs below. Only straddle type fixtures are suitable for ADA-compliance.

2) Rim Height: [20 inches (508 mm), straddle configuration,] [24 inches (610 mm)] <Insert value> above finished floor.

3) Rim Height for ADA-Compliant Installation: [17 inches (432 mm)] <Insert value> above finished floor, straddle fixture configuration.

Retain one of first two subparagraphs below. Basis-of-design product manufacturer's standard is 3/4 NPT inlet connection. 6 ft. and 8 ft. trough urinals require 1-1/2 NPT inlet.

e. Water Inlet Connection:

1) Washout: 3/4 NPT, [top] [back] spud.

2) Washout Trough: 1-1/2 NPT, [top] [back] spud.

f. Back Outlet Connection: NPS 2 (DN 50) P-trap with O-ring adapter for 2 inch NCT connection.

Select 0.125 or 0.5 gallon per flush in "Water Consumption, Washout Fixtures" Subparagraph below when WaterSense standard applies. Only Acorn Engineering model 2158 can comply with WaterSense standard.

g. Water Consumption, Washout Fixtures: [0.125 gal (0.5 L)] [0.5 gal (1.9 L), high-efficiency] [1.0 gal (3.8 L)] [1.5 gal (5.7 L)] per flush.

h. Supply Pressure: 25 psig (172 kPa), minimum.

i. Drain: Beehive dome strainer, tamper-resistant fasteners.

j. Mounting: Installed from fixture side to reinforced wall with backing that matches fixture-mounting locations.

Coordinate "Flushometer Valve" Subparagraph below with "Urinal Flushometer Valves" Article.

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.
a. Flushometer-Valve Type: [Mechanical] [Hydraulic] [Electronic].

b. Flushometer-Valve Accessories: [Top supply flush valve cover] [Wall supply flush valve access panel].

c. Inlet Connection: NPS 3/4 (DN 19), [top] [back] spud.

Retain "Power" Subparagraph below for electronically controlled flushometer valves.

d. Power: 120 Vac.

All accessories listed in "Accessories" Subparagraph below are optional. Retain only subparagraphs needed to comply with Project design requirements. Waste adapters shown enable Installer to connect waste from fixture side of wall.

4. Accessories:

a. Flush valve through wall connection.

b. Waste adapter to 2 inch NCT, female thread.

c. Waste adapter to 2 inch NPT, male thread.

d. Waste adapter to 2 inch NPT, female thread.

e. Waste adapter to NPS 2, no-hub.

"Transformer" Subparagraph below is for applications of basis-of-design product manufacturer's products equipped with Acorn's "Model MVC Time-trol" electronic flush valve.

f. Transformer: 120 V ac to 24 V ac.

Copy "Urinals - Wall Hung, Bottom Outlet, Washout" Paragraph below and re-edit for each type of wall-hung, bottom-outlet, washout urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

D. Urinals - Wall Hung, Bottom Outlet, Washout: <Insert drawing designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Briggs Plumbing Products, Inc.

c. Zurn Industries, LLC.

d. <Insert manufacturer's name>.

2. Fixture:

a. Standards: ASME A112.19.2/CSA B45.1 and ASME A112.19.5/CSA B45.15.

b. Material: Vitreous china.

c. Drain: Separate removable chrome-plated dome strainer with chrome-plated, NPS 1-1/2 (DN 40) tailpiece.

d. Strainer or Trapway: Manufacturer's standard strainer and NPS 1-1/2 (DN 40) tailpiece.

e. Inlet Spud Size and Location: NPS 3/4 (DN 20); top.

f. Outlet Size and Location: NPS 1-1/2 (DN 40); bottom.

g. Color: [White] <Insert color>.

In "Water Consumption" Subparagraph below, not all manufacturers offer each option. Consult manufacturers.

h. Water Consumption: [0.125 gpf (0.5 Lpf)] [0.25 gpf (1.0 Lpf)] [0.5 gpf (1.9 Lpf)] [1.0 gpf (3.8 Lpf)].

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.

4. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap:

1) Size: NPS 1-1/2 (DN 50).

Retain "Material, Chrome Plated" or "Material, Stainless Steel" Subparagraph below.

2) Material, Chrome Plated: [Two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick tubular brass wall bend] <Insert trap type>; and chrome-plated brass or steel wall flange.

3) Material, Stainless Steel: Two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick, stainless steel tube to wall; and stainless steel wall flange.

Coordinate "Support" Subparagraph below with "Supports" Article.

5. Support: [Type II urinal carrier] <Insert carrier> with hanger and bearing plates.[ Include rectangular, steel uprights.]

6. Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Copy "Urinals - Wall Hung, Back Outlet, Washdown" Paragraph below and re-edit for each type of wall-hung, back-outlet, washdown urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

E. Urinals - Wall Hung, Back Outlet, Washdown: [Accessible] <Insert drawing designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Kohler Co.

c. <Insert manufacturer's name>.

2. Fixture:

a. Standards: ASME A112.19.2/CSA B45.1 and ASME A112.19.5/CSA B45.15.

b. Material: Vitreous china.

c. Type: Washdown with extended shields.

d. Strainer or Trapway: [Manufacturer's standard strainer] <Insert strainer> with integral trap.

In "Water Consumption" Subparagraph below, not all manufacturers offer each option. Consult manufacturers.

e. Water Consumption: [0.125 gpf (0.5 Lpf)] [0.25 gpf (1.0 Lpf)] [0.5 gpf (1.9 Lpf)] [1.0 gpf (3.8 Lpf)].

First option in "Spud Size and Location" Subparagraph below is available from some manufacturers. Back spud is used with a remote hard-wired, solenoid-actuator, electronic-sensor flushometer valve. Verify availability if required.

f. Spud Size and Location: NPS 3/4 (DN 20), [back] [top].

g. Outlet Size and Location: NPS 2 (DN 50), back.

h. Color: [White] <Insert color>.

3. Flushometer Valve: <Insert urinal flushometer-valve designation from "Urinal Flushometer Valves" Article>.

4. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2 for coupling.

b. Size: NPS 2 (DN 50).

Coordinate "Support" Subparagraph below with "Supports" Article.

5. Support: [Type I urinal carrier] <Insert carrier> with fixture support plates and coupling with seal and fixture bolts and hardware matching fixture.[ Include rectangular, steel uprights.]

6. Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Copy "Urinals - Wall Hung, Bottom Outlet, Washdown" Paragraph below and re-edit for each type of wall-hung, bottom-outlet, washdown urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

F. Urinals, Wall Hung, Bottom Outlet, Washdown: <Insert drawing designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advanced Modern Technologies Corporation - AMTC.

b. American Standard.

c. Kohler Co.

d. Zurn Industries, LLC.

e. <Insert manufacturer's name>.

2. Fixture:

a. Standards: ASME A112.19.1/CSA B45.2 and ASME A112.19.5/CSA B45.15.

b. Material: Enameled cast iron.

c. Size: [36 inches (915 mm)] [48 inches (1219 mm)] [60 inches (1525 mm)] [72 inches (1830 mm)].

d. Drain: Separate removable chrome-plated dome strainer with chrome-plated, NPS 1-1/2 (DN 40) tailpiece.

In "Water Consumption" Subparagraph below, not all manufacturers offer each option. Consult manufacturers.

e. Water Consumption: [0.125 gpf (0.5 Lpf)] [0.25 gpf (1.0 Lpf)] [0.5 gpf (1.9 Lpf)] [1.0 gpf (3.8 Lpf)].
f. Flushing Device: Manufacturer's standard self-closing valve with washdown pipe of length matching fixture.

g. Outlet Size and Location: NPS 1-1/2 (DN 40); bottom.

h. Color: [White] <Insert color>.

3. Flushometer Valve:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Flushing Device: Manufacturer's standard for NPS 1/2 (DN 15) supply; self-closing valve; and washdown pipe of length matching fixture.

4. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap:

1) Size: NPS 1-1/2 (DN 50).

Retain "Material, Chrome Plated" or "Material, Stainless Steel" Subparagraph below.

2) Material, Chrome Plated: [Two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick tubular brass wall bend] <Insert trap type>; and chrome-plated brass or steel wall flange.

3) Material, Stainless Steel: Two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick, stainless steel tube to wall; and stainless steel wall flange.

Coordinate "Support" Subparagraph below with "Supports" Article.

5. Support: [Type I sink carrier] <Insert carrier>.

6. Sink/Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

2.3 WATERLESS URINALS

Copy "Urinals - Wall Hung, Back Outlet, Waterless, Vitreous China" Paragraph below and re-edit for each type of wall-hung, back-outlet, waterless, vitreous-china urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

A. Urinals - Wall Hung, Back Outlet, Waterless, Vitreous China: <Insert drawing designation>. Designed for liquid-trap-seal operation.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Duravit USA, Inc.

b. Falcon Waterfree Technologies.

c. Kohler Co.

d. Waterless Co.

e. Zero Flush.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1, except without water supply.

b. Trap-Seal Method: Proprietary cartridge with liquid seal.

c. Outlet Size and Location: [NPS 2 (DN 50)] <Insert pipe size> flange; back.

d. Trap-Sealing Liquid: Proprietary.

e. Color: [White] <Insert color>.

3. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2 for transition coupling, trap, and waste pipe.

b. Size: [NPS 2 (DN 50)] <Insert pipe size>.

Coordinate "Support" Subparagraph below with "Supports" Article.

4. Support: [Type I urinal carrier] <Insert carrier> with fixture support plates and coupling with seal and fixture bolts and hardware matching fixture.[ Include rectangular, steel uprights.]

5. Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Copy "Urinals - Wall Hung, Back Outlet, Waterless, Plastic" Paragraph below and re-edit for each type of wall-hung, back-outlet, waterless, plastic urinal required.

Insert drawing designation. Use these designations on Drawings to identify each urinal.

B. Urinals - Wall Hung, Back Outlet, Waterless, Plastic: <Insert drawing designation>. Designed for liquid-trap-seal operation.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Waterless Co.

b. <Insert manufacturer's name>.

2. Fixture:

a. Standard: IAPMO Z124/CSA B45.5, except without water supply.

b. Material: [FRP] <Insert material>.

c. Trap-Seal Method: Proprietary cartridge or trap system.

d. Outlet Size and Location: NPS 2 (DN 50); back. Include transition coupling.

e. Trap-Sealing Liquid: Proprietary.

f. Color: [White] <Insert color>.

3. Waste Fitting:

a. Standard: ASME A112.18.2/CSA B125.2 for transition coupling, trap, and waste pipe.

b. Size: [NPS 2 (DN 50)] <Insert pipe size>.

Coordinate "Support" Subparagraph below with "Supports" Article.

4. Support: [Metal plate in wall] [Wood blocking in wall] [Type I urinal carrier with fixture support plates and coupling with seal and fixture bolts and hardware matching fixture] <Insert carrier>.[ Include rectangular, steel uprights.]

5. Urinal Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

2.4 URINAL FLUSHOMETER VALVES

Caution: Flushometer-valve consumption must match the water consumption of the urinal.

Copy "Lever-Handle, Diaphragm Flushometer Valves" Paragraph below and re-edit for each type of lever-handle, diaphragm flushometer valve required.

Insert designation. Use these designations to identify each valve.

A. Lever-Handle, Diaphragm Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advanced Modern Technologies Corporation - AMTC.

b. American Standard.

c. Delany Products.

d. Gerber Plumbing Fixtures LLC.
e. Zurn Industries, LLC.

f. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

10. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

11. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Solenoid-Actuator, Diaphragm Flushometer Valves" Paragraph below and re-edit for each type of solenoid-actuator, diaphragm flushometer valve required.

Insert designation. Use these designations to identify each valve.

B. Solenoid-Actuator, Diaphragm Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Delany Products.

b. Stern Engineering Ltd.
c. Zurn Industries, LLC.

d. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Actuator: Solenoid complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

10. Trip Mechanism: [Battery-powered] [Hard-wired] electronic sensor complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

11. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

12. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

13. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Motor-Actuator, Diaphragm Flushometer Valves" Paragraph below and re-edit for each type of motor-actuator, diaphragm flushometer valve required.

Insert designation. Use these designations to identify each valve.

C. Motor-Actuator, Diaphragm Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advanced Modern Technologies Corporation - AMTC.

b. Zurn Industries, LLC.

c. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Actuator: Geared-motor actuator complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

10. Trip Mechanism: [Battery-powered] [Hard-wired] electronic sensor complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

11. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

12. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

13. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Hydraulic-Actuator, Push-Button, Diaphragm Flushometer Valves" Paragraph below and re-edit for each type of hydraulic-actuator, push-button, diaphragm flushometer valve required.

Insert designation. Use these designations to identify each valve.

D. Hydraulic-Actuator, Push-Button, Diaphragm Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Delany Products.

b. Zurn Industries, LLC.

c. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

10. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

11. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Lever-Handle, Piston Flushometer Valves" Paragraph below and re-edit for each type of lever-handle, piston flushometer valve required.

Insert designation. Use these designations to identify each valve.

E. Lever-Handle, Piston Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Delany Products.

c. Kohler Co.

d. TOTO USA, INC.

e. Zurn Industries, LLC.

f. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

10. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

11. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Hard-Wired, Solenoid-Actuator, Piston Flushometer Valves" Paragraph below and re-edit for each type of hard-wired, solenoid-actuator, piston flushometer valve required.

Insert designation. Use these designations to identify each valve.

F. Hard-Wired, Solenoid-Actuator, Piston Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. TOTO USA, INC.

b. Zurn Industries, LLC.

c. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Actuator: Solenoid complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

10. Trip Mechanism: Hard-wired electronic sensor complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

11. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

12. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

13. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

Copy "Battery-Powered, Solenoid-Actuator, Piston Flushometer Valves" Paragraph below and re-edit for each type of battery-powered, solenoid-actuator, piston flushometer valve required.

Insert designation. Use these designations to identify each valve.

G. Battery-Powered, Solenoid-Actuator, Piston Flushometer Valves: <Insert designation>.
Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Delany Products.

c. Hydrotek International, Inc.
d. Kohler Co.

e. TOTO USA, INC.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Standard: ASSE 1037/ASME 112.1037/CSA B125.37.

3. Minimum Pressure Rating: 125 psig (860 kPa).

4. Features: Include integral check stop and backflow-prevention device.

5. Material: Brass body with corrosion-resistant components.

6. Exposed Flushometer-Valve Finish: Chrome plated.

Retain "Panel Finish" Subparagraph below for concealed flushometer-valve components.

7. Panel Finish: Chrome plated or stainless steel.

8. Style: [Exposed] [Concealed].

9. Actuator: Solenoid complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

10. Trip Mechanism: Battery-powered electronic sensor complying with UL 1951; listed and labeled as defined in NFPA 70, by a qualified testing agency; and marked for intended location and application.

11. Consumption: [0.5 gal. (1.9 L)] [1.0 gal. (3.8 L)] <Insert value> per flush.

12. Minimum Inlet: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)].

13. Minimum Outlet: [NPS 3/4 (DN 20)] [NPS 1-1/4 (DN 32)].

2.5 SUPPORTS

A. Type I Urinal Carrier:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Jay R. Smith Mfg Co; a division of Morris Group International.

b. Josam Company.

c. MIFAB, Inc.

d. Wade Drains.

e. WATTS.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Standard: ASME A112.6.1M.

B. Type II Urinal Carrier:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Jay R. Smith Mfg Co; a division of Morris Group International.

b. Josam Company.

c. MIFAB, Inc.

d. Wade Drains.

e. WATTS.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Standard: ASME A112.6.1M.

C. Type I Sink Carrier:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Jay R. Smith Mfg Co; a division of Morris Group International.

b. Josam Company.

c. MIFAB, Inc.

d. Wade Drains.

e. WATTS.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Standard: ASME A112.6.1M.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in of water supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before urinal installation.

B. Examine walls and floors for suitable conditions where urinals will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Urinal Installation:

1. Install urinals level and plumb in accordance with rough-in drawings.

2. Install wall-hung, back-outlet urinals onto waste fitting seals and attached to supports.

3. Install wall-hung, bottom-outlet urinals with tubular waste piping attached to supports.

Indicate on Drawings those urinals that are required to be accessible.

4. Install accessible, wall-mounted urinals at mounting height for the handicapped/elderly, in accordance with ICC A117.1.

5. Install trap-seal liquid in waterless urinals.

B. Support Installation:

1. Install supports, affixed to building substrate, for wall-hung urinals.

2. Use off-floor carriers with waste fitting and seal for back-outlet urinals.

3. Use carriers without waste fitting for urinals with tubular waste piping.

4. Use chair-type carrier supports with rectangular steel uprights for accessible urinals.

C. Flushometer-Valve Installation:

1. Install flushometer-valve water-supply fitting on each supply to each urinal.

2. Attach supply piping to supports or substrate within pipe spaces behind fixtures.

3. Install lever-handle flushometer valves for accessible urinals with handle mounted on open side of compartment.

4. Install fresh batteries in battery-powered, electronic-sensor mechanisms.

D. Wall Flange and Escutcheon Installation:

1. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations.

2. Install deep-pattern escutcheons if required to conceal protruding fittings.

3. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

Retain "Joint Sealing" Paragraph below if sealants are not specified in Section 079200 "Joint Sealants."

E. Joint Sealing:

1. Seal joints between urinals and walls and floors using sanitary-type, one-part, mildew-resistant silicone sealant.

2. Match sealant color to urinal color.

3. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

3.3 PIPING CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect urinals with water supplies and soil, waste, and vent piping. Use size fittings required to match urinals.

B. Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

C. Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

D. Where installing piping adjacent to urinals, allow space for service and maintenance.

3.4 ADJUSTING

A. Operate and adjust urinals and controls. Replace damaged and malfunctioning urinals, fittings, and controls.

B. Adjust water pressure at flushometer valves to produce proper flow.

C. Install fresh batteries in battery-powered, electronic-sensor mechanisms.

3.5 CLEANING AND PROTECTION

A. Clean urinals and fittings with manufacturers' recommended cleaning methods and materials.

B. Install protective covering for installed urinals and fittings.

C. Do not allow use of urinals for temporary facilities unless approved in writing by Owner.

END OF SECTION 224213.16
	COMMERCIAL URINALS
	224213.16 - 1



