Copyright 2015 AIA
MasterSpec Full Length
03/15 (PM 08/21)

PRODUCT MASTERSPEC LICENSED BY DELTEK, INC. TO ACORN ENGINEERING; MORRIS GROUP INTERNATIONAL.

Copyright 2015 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA
This Product MasterSpec Section is licensed by Deltek, Inc. to Acorn Engineering; Morris Group International ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit Project requirements.

For more information, contact Acorn Engineering; Morris Group International, 15125 Proctor Ave., City of Industry, CA 91746; Phone: (626) 336-4561 | (800) 488-8999; Website: www.acorneng.com; Email: dyoung@acorneng.com.

For information about MasterSpec, contact Deltek at (800) 424-5080 or visit masterspec.com

 HYPERLINK "http://" .

SECTION 224233 - WASH FOUNTAINS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com.>
Revise this Section by deleting and inserting text to meet Project-specific requirements.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Circular wash fountains.

2. Semicircular wash fountains.

3. Corner wash fountains.

4. Elliptical wash fountains.

5. Linear wash fountains.

B. Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 224216.13 "Commercial Lavatories."

2. Section 224216.16 "Commercial Sinks."

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

1. Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for wash fountains.

2. Include rated capacities, operating characteristics,[ electrical characteristics,] and furnished specialties and accessories.

B. Sustainable Design Submittals:

"Product Data" Subparagraph below applies to LEED (including v4), IgCC, ASHRAE 189.1, and Green Globes for fixtures and equipment that consume water.

1. Product Data: For water consumption.
Retain "Shop Drawings" Paragraph below if fixtures or equipment includes wiring.

C. Shop Drawings: For each type of wash fountain.

1. Include plans, elevations, sections, and [mounting] [attachment] details.

2. Include details of equipment assemblies. Indicate dimensions, weights, loads, required clearances, method of field assembly, components, and location and size of each field connection.

3. Include diagrams for power, signal, and control wiring.

Retain subparagraph below when units with painted finish are required.

4. Color samples.

1.3 CLOSEOUT SUBMITTALS

A. Operation and Maintenance Data: For wash fountains and components to include in operation and maintenance manuals.

1.4 MAINTENANCE MATERIAL SUBMITTALS

A. Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Faucet Washers and O-Rings: Equal to [10] <Insert number> percent of quantity of each type and size installed.

2. Faucet Cartridges and O-Rings: Equal to [5] <Insert number> percent of quantity of each type and size installed.

PART 2 -  PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."
Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Do not use the same plumbing fixture designation as is used for a plumbing fixture in another plumbing Section.

2.1 PRECAST-TERRAZZO, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains" Paragraph below and re-edit for each type of circular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each precast-terrazzo, circular wash fountain.

A. Wash Fountains <Insert drawing designation>: Precast-terrazzo, circular receptor.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by the following:

a. Bradley Corporation.

b. <Insert manufacturer's name>.

2. Standard: IAPMO IGC 156.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: IAPMO PS 99 for precast-terrazzo receptor.

b. Nominal Diameter: [36 to 39 inches (914 to 990 mm)] [39 inches (990 mm)] [42 inches (1067 mm)] [54 inches (1372 mm)].

c. Height to Rim: 29 inches (737 mm) above floor.

d. Color or Finish: [Not applicable] <Insert if required>.

e. Drain: Grid with [NPS 2 (DN 50)] <Insert size> tailpiece.

5. Spray Head:

a. Material: Stainless steel.

b. Number of Wash Stations: [Five] [Six] [Eight].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

d. Control: [Collective, foot-rail] [Individual foot-pedal] [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] actuation with [pressure-balancing] [thermostatic] mixing valve complying with ASSE 1016 and having check stops; comply with NSF 61 Annex G.

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

e. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Verify "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option, if retained in "Liquid-Soap Dispensers" Subparagraph below, is available from listed manufacturers and for type of wash fountain required. Typically, sensor-operated soap dispensers are battery powered; however, they require hardwiring if the nozzles are hardwired.

6. Liquid-Soap Dispensers: [Manual] [Hardwired, sensor actuated] [Battery powered, sensor actuated] [Not required], for each wash station.

Retain "Sensor" Subparagraph below if retaining "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option in "Liquid-Soap Dispensers" Subparagraph above.

a. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

7. Pedestal: [Manufacturer's standard] [Painted or coated steel] [Stainless steel] with access panel.

8. Supply Fittings:

a. Piping: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] copper tubing.

b. Valves: Shutoff valve on each supply.

c. Supply Piping: From [bottom] [top].

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: [NPS 2 (DN 50)].

c. Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

10. Shroud: Stainless steel of size to cover supply and vent piping.

11. Mounting: Floor bracket.

2.2 SOLID-SURFACE, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains" Paragraph below and re-edit for each type of circular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each solid-surface, circular wash fountain.

A. Wash Fountains <Insert drawing designation>: Solid-surface, circular receptor.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Bradley Corporation.

b. Intersan Manufacturing Company.

c. Willoughby Industries.

d. <Insert manufacturer's name>.

2. Standard: IAPMO IGC 156.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ICPA SS-1 for solid-surface receptor.

b. Nominal Diameter: [36 to 39 inches (914 to 990 mm)] [39 inches (990 mm)] [42 inches (1067 mm)] [54 inches (1370 mm)].

c. Height to Rim: 34 inches (864 mm) above floor.

d. Color or Finish: [Not applicable] <Insert if required>.

e. Drain: Grid with [NPS 2 (DN 50)] <Insert size> tailpiece.

5. Spray Head:

a. Material: Stainless steel.

b. Number of Wash Stations: [Five] [Six] [Eight].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

d. Control: [Collective, foot-rail] [Individual foot-pedal] [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] actuation with [pressure-balancing] [thermostatic] mixing valve complying with ASSE 1016 and having check stops; comply with NSF 61 Annex G.

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

e. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Verify "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option, if retained in "Liquid-Soap Dispensers" Subparagraph below, is available from listed manufacturers and for type of wash fountain required. Typically, sensor-operated soap dispensers are battery powered; however, they require hardwiring if the nozzles are hardwired.

6. Liquid-Soap Dispensers: [Manual] [Hardwired, sensor actuated] [Battery powered, sensor actuated] [Not required], for each wash station.

Retain "Sensor" Subparagraph below if retaining "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option in "Liquid-Soap Dispensers" Subparagraph above.

a. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

7. Pedestal: [Manufacturer's standard] [Painted or coated steel] [Stainless steel] with access panel.

8. Supply Fittings:

a. Piping: [NPS 3/4 (DN 20)] [NPS 1 (DN 25)] copper tubing.

b. Valves: Shutoff valve on each supply.

c. Supply Piping: From [bottom] [top].

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: [NPS 2 (DN 50)].

c. Vent Piping: [Not required] [NPS 1-1/2 (DN 40) to ceiling].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

10. Shroud: Stainless steel of size to cover supply and vent piping.
11. Mounting: Floor bracket.

2.3 STAINLESS STEEL, CIRCULAR WASH FOUNTAINS

Caution: These fixtures require a trap with waste and vent connections for installation in the base and vertical vent piping through top of unit, or they may be installed with trap and vent under the floor below the fixture.

Copy "Wash Fountains" Paragraph below and re-edit for each type of circular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each stainless steel, circular wash fountain.

A. Wash Fountains <Insert drawing designation>: Stainless steel, circular receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.
Retain Acorn Engineering model 3504 below for wash fountain with four wash stations.

Retain model 3506 for wash fountain with six wash stations. Retain model 3506ADA for wash fountain with six wash stations and ADA-compliant configuration.

Retain model 3506DBS and model 3508DBS for wash fountain with six or eight wash stations, respectively, with deep-bowl style receptor, non-ADA compliant.

Retain model 3508 for wash fountain with eight wash stations. Retain model 3508ADA for wash fountain with eight wash stations and ADA-compliant configuration.

Retain model 3516ADA for wash fountain with six wash stations and ADA-compliant configuration.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International, Wash-ware circular stainless steel, model [3504] [3506] [3506ADA] [3506DBS] [3508] [3508ADA] [3508DBS] [3516ADA]; <Insert product name or designation> or comparable product by one of the following:
a. Bradley Corporation.

b. Intersan Manufacturing Company.

c. Willoughby Industries.

d. <Insert manufacturer's name>.

2. Standard: ASME A112.19.3/CSA B45.4.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant fixtures are specified.

b. ADA Standards for Accessible Design.

In the "Nominal Diameter" Subparagraph below, retain first option when Acorn Engineering model 3516ADA is specified. When wheelchair access for either six or eight wash station fixture is required, retain second option.
c. Nominal Diameter: [38-1/4 inches (972 mm)] [54 inches (1372 mm)].

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design, and when ADA compliance is not required.

d. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

In "Depth" Subparagraph below, retain second option when fixture with deep bowl style receptor is specified.

e. Depth: [Manufacturer's standard] [9 inches (229 mm)].

f. Material: Stainless steel.

In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

g. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart.].
h. Drain: Dome strainer.

5. Spray Head:

a. Material: Stainless steel.

b. Number of Wash Stations: [Four] [Six] [Eight].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.
Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Certain ADA-compliant models not available with push-button or foot controls.
d. Control: [Individual foot-button] [Individual manual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering] actuation.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design. The soap dispenser options below do not comply with ADA requirements.
6. Soap Dispensers: [Powder soap] [Liquid or lotion soap] [Metering liquid soap pump].

In "Pedestal Material and Finish" Subparagraph below, black vinyl-clad galvanized steel is manufacturer's standard. Retain other option when required by Project design.
7. Pedestal Material and Finish: [Black vinyl-clad galvanized steel] [Stainless steel, No. 4 satin finish].

8. Supply Fittings:

a. Supply Piping: From [bottom] [top].

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: [NPS 2 (DN 50)].

c. Vent Piping: [Off-drain] [Through-fixture].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

10. Shroud: Stainless steel of size to cover supply and vent piping.
Retain "Paper Towel Dispenser" Subparagraph below if required by Project design.

11. Paper Towel Dispenser: [C-Fold or Multi-Fold] [Single Fold].
12. Accessories:
a. Hose bibb.

b. Pipe column brace.

13. Mounting: On-floor with floor-mounting flange on wash fountain base interior.
2.4 PRECAST-TERRAZZO, SEMICIRCULAR WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of semicircular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each precast-terrazzo, semicircular wash fountain.

A. Wash Fountains <Insert drawing designation>: [Off] [On]-floor, precast-terrazzo, semicircular receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3601 below for wash fountain with one wash station, ADA-compliant configuration.

Retain model 3603 for wash fountain with three wash stations, ADA-compliant configuration.

Retain model 3604 for wash fountain with four wash stations, ADA-compliant configuration.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International, Wash-ware, model [3601] [3603] [3604]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. <Insert manufacturer's name>.

2. Standard:
a. IAPMO IGC 156.

b. ADA Standards for Accessible Design.
3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: IAPMO PS 99 for precast-terrazzo receptor.

b. Configuration: Back wall, semicircular.
c. Material: Precast terrazzo.

In "Mounting" Subparagraph below, the off-floor option is available for one-station units only. The standard configuration is pedestal, on-floor mounting.

d. Mounting: [On-floor] [Off-floor].
e. Height to Rim: 33-1/2 to 34 nches (851 to 864 mm) above floor.

f. Color or Finish: <Insert color selection from manufacturer's standard selection>.

g. Drain: Grid strainer.

5. Spray Head:

a. Material: Precast terrazzo.

b. Number of Wash Stations: [One] [Three] [Four].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.
Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Certain ADA-compliant models not available with push-button or foot controls.
d. Control: [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering].

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

In "Pedestal" Subparagraph below, retain "Stainless steel" option when Acorn Engineering model 3601 is specified.
6. Pedestal: [Stainless steel] [Terrazzo with stainless steel access panel secured by vandal-resistant fasteners].

Retain "Soap Dispensers" Subparagraph below when required for Project design.

7. Soap Dispensers: Metering liquid soap pump.

8. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.

9. Off-Floor Mounting: In accordance with manufacturer's written instructions.

2.5 SOLID-SURFACE, SEMICIRCULAR WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of semicircular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each solid-surface, semicircular wash fountain.

A. Wash Fountains <Insert drawing designation>: On-floor, solid-surface, semicircular receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3633 option for back-wall wash fountain with three wash stations.

Retain model 3644 for back-wall wash fountain with four wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware Corterra, model [3633] [3644]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Intersan Manufacturing Company.

c. Willoughby Industries.

d. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.
d. GREENGUARD Certified Low Emitting Material.
e. ANSI/ICPA SS-1 for solid-surface bowls.
f. ADA Standards for Accessible Design.
3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: CSA B45.5/IAPMO Z124.

b. Fixture mounting: [On-floor] [Off-floor].
c. Configuration: [Back wall, semicircular] [Corner].
In "Nominal Diameter" Subparagraph below, retain first option for Acorn Engineering Model 3633. Retain second option for Model 3644.

d. Nominal Diameter: [36 inches (914 mm)] [46 inches (1168 mm)] <Insert dimension>.

In "Height to Rim" Subparagraph below, manufacturer's standard product is 34 inches (864 mm) above floor. Retain first option for "Junior" height when required for Project design and when ADA compliance is not required.

e. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

f. Color or Finish: <Insert color selection from manufacturer's standard selection>.

g. Drain: Grid with 1-1/2-inch (40-mm) OD tailpiece.

5. Spray Head:

a. Material: Solid surface.

b. Number of Wash Stations: [Three] [Four].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

d. Control: [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering] actuation.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design. 

6. Pedestal: Solid surface material with stainless steel access panel secured by vandal-resistant fasteners.

7. Soap Dispensers: Metering liquid soap pump.

8. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.

2.6 STAINLESS STEEL, SEMICIRCULAR WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of semicircular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each stainless steel, semicircular wash fountain.

A. Wash Fountains <Insert drawing designation>: [Off] [On]-floor, stainless steel, semicircular receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering Model 3422 option for semicircular wash fountain with two wash stations. Retain Model 3422ES for semicircular wash fountain with two wash stations and contoured base.

Retain Model 3423 for semicircular wash fountain with three wash stations. Retain Acorn Engineering Model 3423ES for semicircular wash fountain with three wash stations and contoured base.

Retain Model 3423ADA for ADA-compliant semicircular wash fountain with three wash stations. Retain Model 3423ADAES for ADA-compliant semicircular wash fountain with three wash stations and contoured base.

Retain Model 3424ADA for ADA-compliant semicircular wash fountain with four wash stations. Retain Model 3424ADAES for ADA-compliant semicircular wash fountain with four wash stations and contoured base.

Retain Model 3543 for on-floor semicircular wash fountain with three wash stations. Retain Model 3543DBS for on-floor semicircular wash fountain with three wash stations and deep bowl receptor.

Retain Model 3544 for on-floor semicircular wash fountain with four wash stations. Retain Model 3544DBS for non-ADA-compliant, on-floor semicircular wash fountain with four wash stations and deep bowl receptor.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware stainless steel model [3422] [3422ES] [3423] [3423ADA] [3423ADAES] [3423ES] [3424ADA] [3424ADAES] [3543] [3543DBS] [3544] [3544DBS]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Intersan Manufacturing Company.

c. Willoughby Industries.

d. <Insert manufacturer's name>.

2. Standard:

a. ASME A112.19.3/CSA B45.4.

Retain first subparagraph below for ADA Standards only when ADA-compliant fixtures are specified.

b. ADA Standards for Accessible Design.
3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

b. Nominal Diameter: [30 inches (762 mm)] [38 inches (965 mm)] [54 inches (1372 mm)].

In "Height to Rim" Subparagraph below, manufacturer's standard product is 34 inches (864 mm) above floor. Retain first option for "Junior" height when required for Project design and when ADA compliance is not required.

c. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

In "Depth" Subparagraph below, retain second option when fixture with deep bowl style receptor is specified.

d. Depth: [Manufacturer's standard] [9 inches (229 mm)]. 

In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.
e. Finish or Color: [No. 4 satin finish.] [Powder coat; select color from manufacturer's standard color chart.]
f. Drain: Dome strainer.

5. Spray Head:

a. Material: Stainless steel or integral part of receptor back.

b. Number of Wash Stations: [Two] [Three] [Four].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Certain ADA-compliant models not available with push-button or foot controls.
d. Control: [Individual foot-button] [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering] actuation.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design. The soap dispenser options below do not comply with ADA requirements.

6. Soap Dispensers: [Powder soap] [Liquid or lotion soap pump] [Metering liquid soap pump].

In "Pedestal Material and Finish" Subparagraph below, black vinyl-clad galvanized steel is manufacturer's standard. Retain other option when required by Project design.

7. Pedestal Material and Finish: [Black vinyl-clad galvanized steel] [Stainless steel, No. 4 satin].
8. Supply Fittings:

a. Supply Piping: From [bottom] [top] [wall].

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.

In "Trap and Drain Piping" Subparagraph below, retain first option when fixture water supply is through-wall configuration.

c. Trap and Drain Piping: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

d. Vent Piping: [Off-drain] [Through-fixture]. 

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

10. Shroud: Stainless steel of size to cover supply and vent piping.

Retain "Paper Towel Dispenser" Subparagraph below if required by Project design.

11. Paper Towel Dispenser: [C-Fold or Multi-Fold] [Single Fold].

In "Accessories" Subparagraph below, retain only subparagraphs needed to comply with Project design requirements.

12. Accessories:

a. Hose bibb [left side] [right side].
b. Pipe column brace.
c. Column braced to backsplash.
d. Less backsplash.
13. Mounting: [Off-floor, on reinforced wall] [On-floor, with wall- and floor-mounting flanges on wash fountain base interior].

2.7 PRECAST-TERRAZZO, CORNER WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of corner wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each precast-terrazzo, corner wash fountain.

A. Wash Fountains <Insert drawing designation>: Precast-terrazzo, corner receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering Model 3612 option for corner wash fountain with two wash stations.

Retain Model 3613 for corner wash fountain with three wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware Corner Terrazzo model [3612] [3613]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. <Insert manufacturer's name>.

2. Standard: IAPMO IGC 156.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: IAPMO PS 99 for precast-terrazzo receptor.

b. Nominal Side Width: [26-3/4 inches (680 mm)] [37 inches (940 mm)].

c. Height to Rim: [34 inches (864 mm)] <Insert dimension> above floor.

d. Color or Finish: <Insert color from manufacturer's standard selection>.

e. Drain: Grid strainer.

5. Spray Head:

a. Material: Precast terrazzo.

b. Number of Wash Stations: [Two] [Three].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.
Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Certain ADA-compliant models are not available with push-button or foot controls.
d. Control: [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering].

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

g. Pedestal: Terrazzo with stainless steel access panel secured with vandal-resistant fasteners.

Retain "Soap Dispensers" Subparagraph below when required for Project design.
6. Soap Dispensers: Metering liquid soap pump, for each wash station.

7. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.
8. Mounting: In accordance with manufacturer's written instructions.

2.8 SOLID-SURFACE, CORNER WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of corner wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each solid-surface, corner wash fountain.

A. Wash Fountains <Insert drawing designation>: Solid-surface, corner receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware Corterra model 3622; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard: IAPMO IGC 156.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ICPA SS-1 for solid-surface receptor.

b. Nominal Side Width: [27 inches (686 mm)] [37 inches (940 mm)].

c. Height to Rim: [34 inches (864 mm)] <Insert dimension> above floor.

d. Color or Finish: [Not applicable] <Insert if required>.

e. Drain: Grid with NPS 1-1/2 (DN 40) tailpiece.

5. Spray Head:

a. Material: Stainless steel or integral part of receptor back.

b. Number of Wash Stations: Two.

c. Spray Nozzles: Chrome-plated brass or stainless steel complying with NSF 61 Annex G and ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

d. Control: [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] actuation with [pressure-balancing] [thermostatic] mixing valve complying with ASSE 1016 and having check stops; comply with NSF 61 Annex G.

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

e. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Verify "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option, if retained in "Liquid-Soap Dispensers" Subparagraph below, is available from listed manufacturers and for type of wash fountain required. Typically, sensor-operated soap dispensers are battery powered; however, they require hardwiring if the nozzles are hardwired.

6. Liquid-Soap Dispensers: [Manual] [Hardwired, sensor actuated] [Battery powered, sensor actuated] [Not required], for each wash station.

Retain "Sensor" Subparagraph below if retaining "Hardwired, sensor actuated" or "Battery powered, sensor actuated" option in "Liquid-Soap Dispensers" Subparagraph above.

a. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

7. Supply Fittings:

a. Piping: [NPS 1/2 (DN 15)] [NPS 3/4 (DN 20)] copper tubing.

b. Valves: Shutoff valve on each supply.

c. Supply Piping: From wall.

8. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40).

9. Off-Floor Mounting: Wall bracket and ASME A112.6.1M, Type II urinal carrier.

2.9 STAINLESS STEEL, CORNER WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of corner wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each precast-terrazzo, corner wash fountain.

A. Wash Fountains <Insert drawing designation>: Stainless steel, corner receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.
Retain Acorn Engineering Model 3442 option for corner wash fountain with two wash stations. Retain Model 3442ES for corner wash fountain with two wash stations and contoured base.

Retain 3443ADA for ADA-compliant corner wash fountain with three wash stations. Retain Model 3443ADAES for ADA-compliant corner wash fountain with three wash stations and contoured base.

Retain Model 3562 for on-floor corner wash fountain with two wash stations. Retain Acorn Engineering Model 3562DBS for on-floor corner wash fountain with two wash stations and deep bowl receptor.

Retain Model 3563 for on-floor corner wash fountain with three wash stations. Retain model 3563DBS for on-floor corner wash fountain with three wash stations and deep bowl receptor.
1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware stainless steel, model [3442] [3442ES] [3443ADA] [3443ADAES] [3562] [3562DBS] [3563] [3563DBS]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. <Insert manufacturer's name>.

2. Standard: 
a. ASME A112.19.3/CSA B45.4.

Retain first subparagraph below for ADA Standards only when ADA-compliant fixtures are specified.

b. ADA Standards for Accessible Design.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

b. Nominal Side Width: 37 inches (940 mm).

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.

c. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

In "Depth" Subparagraph below, retain second option when fixture with deep bowl style receptor is specified.

d. Depth: [Manufacturer's standard] [9 inches (229 mm)]. 

In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

e. Finish or Color: [No. 4 satin finish.] [Powder coat; select color from manufacturer's standard color chart.]
f. Drain: Dome strainer.

5. Spray Head:

a. Material: Stainless steel or integral part of receptor back.

b. Number of Wash Stations: [Two] [Three].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Certain ADA-compliant models not available with push-button or foot controls.
d. Control: [Individual foot-button] [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering] actuation.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design. The soap dispenser options below do not comply with ADA requirements.
6. Soap Dispensers: [Powder soap] [Liquid or lotion soap] [Metering liquid soap pump].

In "Pedestal Material and Finish" Subparagraph below, black vinyl-clad galvanized steel is manufacturer's standard. Retain other option when required by Project design.

7. Pedestal Material and Finish: [Black, vinyl-clad galvanized steel] [Stainless steel, No. 4 satin finish].

8. Supply Fittings:

a. Supply Piping: From [Wall] [Bottom] [Top].

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

In "Trap and Drain Piping" Subparagraph below, retain first option when fixture water supply is through-wall configuration.

b. Trap and Drain Piping: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

c. Vent Piping: [Off-drain] [Through-fixture].

Retain "Shroud" Subparagraph below if required to conceal vertical supply and vent piping.

10. Shroud: Stainless steel of size to cover supply and vent piping.

Retain "Paper Towel Dispenser" Subparagraph below if required by Project design.

11. Paper Towel Dispenser: [C-Fold or Multi-Fold] [Single Fold].

In "Accessories" Subparagraph below, retain only subparagraphs needed to comply with Project design requirements.

12. Accessories:

a. Hose bibb [left side] [right side].
b. Pipe column brace.
c. Column braced to backsplash.
d. Less backsplash.
13. Mounting: [Off-floor, on reinforced wall] [On-floor, with wall- and floor-mounting flanges on wash fountain base interior].

2.10 SOLID-SURFACE, LINEAR WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of linear wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each solid-surface, linear (side-by-side) wash fountain.

A. Wash Fountains <Insert drawing designation>: Solid-surface, straight-front, linear (side-by-side) receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3751 option for ADA-compliant, wall-mounted fixture with one wash station.

Retain model 3752 for ADA-compliant, wall-mounted fixture with two wash stations at different heights.

Retain model 3753 for ADA-compliant, wall-mounted fixture with three wash stations at two different heights.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Acorn Meridian Corterra Hi-Lo Dual Basin [3751] [3752] [3753]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.
d. GREENGUARD Certified Low Emitting Material.

e. ANSI/ICPA SS-1 for solid-surface bowls.

f. ADA Standards for Accessible Design.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Bowl(s) and Counter:

a. Standard: CSA B45.5/IAPMO Z124.

b. Counter: Solid surface material with drip-resistant edges and integral overflow.

c. Height to ADA Station Rim: [34 inches (864 mm)] <Insert dimension> above floor.

d. Deck Configuration:

Retain one of first two subparagraphs below. When first subparagraph is retained, retain only one of three options.

1) High deck on [left-hand] [center] [right-hand] wash station.

Retain first subparagraph below only when Acorn Engineering Model 3751 is specified.

2) Standard.

e. Color: Select color from manufacturer's standard color chart. Submit color samples for approval.

f. Number of Receptors: [One] [Two] [Three].

g. Drain: Grid strainer with 1-1/2-inch (40-mm) OD close elbow and tailpiece, each bowl.

5. Mounting: Off-floor.

6. Enclosure: 

a. Material: Type 304 stainless steel, secured with vandal-resistant fasteners.

b. Finish: [No. 4 satin finish] [Powder-coat. Select color from manufacturer's standard color chart].

7. Faucets:

a. Standards: ASME A112.18.1/CSA B125.1 and NSF 61 Annex G.

b. Type: Manufacturer's standard, chrome-plated solid brass, each bowl.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Retain first option to specify manufacturer's standard product.
c. Control: [Less spout and valves. Drill fixture for field-installed faucet with required quantity and spacing of holes] [Hand operation with deck-mounted push-button] [Programmable piezo button with electronic metering] [Hardwired sensor operation with deck-mounted spout] [Battery-powered sensor operation with deck-mounted spout].

Retain "Sensor" Subparagraph below if retaining "Hardwired, sensor operation" or "Battery-powered, sensor operation" option in "Control" Subparagraph above.

d. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

e. Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Soap Dispensers" Subparagraph below when required for Project design. 

8. Soap Dispensers: Metering liquid soap pump for each wash station.

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40), P-trap to accommodate complete fixture.

B. Wash Fountains <Insert drawing designation>: Solid-surface, straight front, linear (side-by-side) receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3761 option for ADA-compliant, wall-mounted fixture with one wash station.

Retain model 3762 for ADA-compliant, wall-mounted fixture with two wash stations.

Retain model 3763 for ADA-compliant, wall-mounted fixture with three wash stations.

Retain model 3764 for ADA-compliant, wall-mounted fixture with four wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Acorn Meridian Corterra Counter-Lav model [3761] [3762] [3763] [3764]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.

d. GREENGUARD Certified Low Emitting Material.

e. ANSI/ICPA SS-1 for solid-surface bowls.

f. ADA Standards for Accessible Design.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Bowl(s) and Counter:

a. CSA B45.5/IAPMO Z124.
In "Counter" Subparagraph below, retain first option for manufacturer's standard product. D-shaped bowls allow for deck-mounted faucet controls.
b. Counter: Solid surface material with drip-resistant front edge, with individual [integral oval bowl with integral overflow] [drop-in, D-shaped stainless steel bowl] [drop-in D-shaped stainless steel bowl with mirror finish].
In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.
c. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

d. Color: Select color from manufacturer's standard color chart. Submit color samples for approval.
e. Number of Receptors: [One] [Two] [Three] [Four].

f. Drain: Cast opening with depression for 1-1/2-inch (DN 40) strainer and tailpiece.

5. Mounting: Off-floor.

6. Valve Control:
Retain either "Faucet" or "Operation" Subparagraph below.

a. Faucet: [Drill fixture for field-installed faucet with required quantity and spacing of holes] [4-inch (102-mm) centerset faucet with gooseneck spout and wrist blade handles].
b. Operation:

1) Control: [Individual deck-mounted push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering].
2) Spout: [Deck-mounted "J" spout] [Deck-mounted "L" spout].

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired, sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

c. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

1) Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].

Retain "Soap Dispensers" Subparagraph below when required for Project design.

7. Soap Dispensers: Metering liquid soap pump for each wash station.

Retain "Accessories" Subparagraph below when required for Project design.

8. Accessories:

a. Extended countertop: [Left hand] [Right hand] <Specify length>.
b. Left side apron.

c. Left side splash.

d. Right side apron.

e. Right side splash.

f. Scalloped enclosure style.

g. Less backsplash.

h. Mounting sleeve for wall carrier.

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

Retain one of the first two subparagraphs below.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40), P-trap to accommodate complete fixture.

In "Trap and Drain Piping with Strainer" Subparagraph below, retain options to coincide with number of wash stations.

c. Trap and Drain Piping with Strainer: [Individual grid strainer and tailpiece with NPS 1-1/2 (DN 40) P-trap for each station] [Individual grid strainers, combined waste piping with NPS 1-1/2 (DN 40) P-trap to serve two wash stations] [Individual grid strainers, combined waste piping with NPS 1-1/2 (DN 40) P-trap to serve three wash stations] [Individual grid strainers with two combined waste piping assemblies for NPS 1-1/2 (DN 40) P-traps serving two wash stations each, to serve four stations total].
C. Wash Fountains <Insert drawing designation>: Solid-surface, curved front, linear (side-by-side) receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3771 option for ADA-compliant, wall-mounted fixture with one wash station.

Retain model 3772 for ADA-compliant, wall-mounted fixture with two wash stations.

Retain model 3773 for ADA-compliant, wall-mounted fixture with three wash stations at two different heights.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Acorn Engineering Meridian Corterra Basin [3771] [3772] [3773]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.

d. GREENGUARD Certified Low Emitting Material.

e. ANSI/ICPA SS-1 for solid-surface bowls.

f. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Bowl(s) and Counter:

a. Standard: CSA B45.5/IAPMO Z124.

b. Spray Head: Solid surface.

In "Counter" Subparagraph below, retain first option for manufacturer's standard product. D-shaped bowls allow for deck-mounted faucet controls.
c. Counter: Solid surface material with integral [15-3/4-inch (400-mm) round bowls and integral overflow] [D-shaped bowl, less spray head].

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.

d. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

e. Color: Select color from manufacturer's standard color chart. Submit color samples for approval.

f. Number of Receptors: [One] [Two] [Three].

g. Drain: Grid strainer, each receptor.

5. Mounting: [Off-floor] [On-floor].

6. Enclosure:

a. Material: Type 304 stainless steel.

In "Material and Finish" Subparagraph below, black vinyl-clad galvanized steel is manufacturer's standard. Retain stainless steel option when required by Project design.

b. Material and Finish: [Black vinyl-clad galvanized steel] [Stainless steel, No. 4 satin finish].

c. Control:

Retain one option from one of first two subparagraphs below. When second subparagraph is retained, coordinate options with D-shaped integral basin.

1) [Hand operation] [Programmable piezo button with electronic metering] [Hardwired sensor operation with deck-mounted faucet] [Battery-powered sensor operation with deck-mounted faucet].

2) [Less spray head and valves. Drill fixture for field-installed faucet with required quantity and spacing of holes] [Less spray head with 4-inch (102-mm) centerset faucet, gooseneck spout and wristblade handles].
Retain "Sensor" Subparagraph below if retaining faucet with "Hardwired, sensor operation" or "Battery-powered, sensor operation" option in "Control" Subparagraph above.

d. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

e. Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].

Retain "Soap Dispensers" Subparagraph below when required for Project design.

7. Soap Dispensers: Metering liquid soap pump for each wash station.

Retain "Accessories" Subparagraph below when required for Project design. Verify that manufacturer of product selected has the required accessories available.

8. Accessories:

a. Extended countertop: [Left hand] [Right hand] <Specify length>.

b. Left side apron, with extended countertop option.

c. Left side splash, with extended countertop option.

d. Right side apron, with extended countertop option.

e. Right side splash, with extended countertop option.

f. Scalloped enclosure style.

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40), P-trap to accommodate complete fixture.

D. Wash Fountains <Insert drawing designation>: Solid-surface, curved front, linear (side-by-side) receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3791 option for ADA-compliant, wall-mounted fixture with one wash station.

Retain model 3792 for ADA-compliant, wall-mounted fixture with two wash stations.

Retain model 3793 for ADA-compliant, wall-mounted fixture with three wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Acorn Meridian Corterra Basin [3791] [3792] [3793]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.

d. GREENGUARD Certified Low Emitting Material.

e. ANSI/ICPA SS-1 for solid-surface bowls.

f. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: CSA B45.5/IAPMO Z124.

b. Counter: Solid surface material with integral individual bowl at each wash station.

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.

c. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

d. Drain: Grid strainer to service entire receptor.

e. Color: Select color from manufacturer's standard color chart. Submit color samples for approval.

5. Spray Head:

a. Material: Solid surface top, Type304 stainless steel faceplate.

b. Number of Wash Stations: [One] [Two] [Three].

c. Control: [Hand operation] [Programmable piezo button with electronic metering] [Hardwired sensor operation] [Battery-powered sensor operation].

Retain "Sensor" Subparagraph below if retaining "Hardwired sensor operation" or "Battery-powered sensor operation" option in "Control" Subparagraph above.

d. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

e. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1.
6. Mounting: Off-floor.

7. Enclosure: 

a. Material: Type 304 stainless steel.

b. Finish: No. 4 satin finish.

Retain "Soap Dispensers" Subparagraph below when required for Project design.
8. Soap Dispensers: Metering liquid soap/lotion pump for each wash station.

Retain "Accessories" Subparagraph below when required for Project design. Verify that manufacturer of product selected has the required accessories available.

9. Accessories:

a. Stainless steel kickplate for [31 inches (787 mm)] [34 inches (864 mm)] <Insert height> height to rim.
b. Less waste piping.

10. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40), P-trap to accommodate complete fixture.

E. Wash Fountains <Insert drawing designation>: Solid-surface, straight front, linear (side-by-side) receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3801 option for ADA-compliant, wall-mounted fixture with one wash station.

Retain model 3802 for ADA-compliant, wall-mounted fixture with two wash stations.

Retain model 3803 for ADA-compliant, wall-mounted fixture with three wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Acorn Meridian Corterra Basin [3801] [3802] [3803]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. CSA B45.5/IAPMO Z124.

b. IAPMO IGC 156.

c. ASME A112.18.1/CSA B125.1.

d. GREENGUARD Certified Low Emitting Material.

e. ANSI/ICPA SS-1 for solid-surface bowls.

f. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Counter: Solid surface material with integral continuous basin.

a. Standard: ICPA SS-1 for solid-surface bowls.

In "Height to Rim" Subparagraph below, standard height is 34 inches. Insert lower height when required by Project design and when ADA compliance is not required.

b. Height to Rim: [31 inches (787 mm)] <Insert height> above floor.

c. Color: Select color from manufacturer's standard color chart. Submit color samples for approval.

d. Number of Receptors: One, for all user wash stations.

e. Number of User Wash Stations: [One] [Two] [Three].

f. Drain: Grid strainer with NPS 1-1/2 (DN 40), close elbow.

5. Mounting: Off-floor.

6. Enclosure:
a. Material: Type 304 stainless steel, hinged.
In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

b. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart].

7. Faucets:

a. Standards: ASME A112.18.1/CSA B125.1 and NSF 61 Annex G.

b. Type: Manufacturer's standard, chrome-plated solid brass, each bowl.

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required.

c. Control: [Less spout and valves. Drill fixture for field-installed faucet with reequired quantity and spacing of holes] [Hand operation with deck-mounted spout and push-button] [Sensor operation with deck-mounted spout] [Battery-powered sensor operation with deck-mounted spout] [Dyson Company Airblade model for sensor-operated, deck-mounted, faucet and air dry].

Retain "Sensor" Subparagraph below if retaining "Sensor operation" or "Battery-powered, sensor-operation" option in "Control" Subparagraph above.

d. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

e. Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].

In "Soap Dispensers" Subparagraph below, verify that sensor-operated soap dispensers are available from selected manufacturer. Typically, sensor-operated soap dispensers are battery powered; however, they require hardwiring if the nozzles are hardwired. 

8. Soap Dispensers: [Metering liquid pump soap] [Hardwired sensor-operated] [Battery-powered sensor-operated] for each user wash station.

Retain "Accessories" Subparagraph below when required for Project design. Verify that manufacturer of product selected has the required accessories available.

9. Accessories:

a. Mounting carrier.
10. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: Brass Plastic 1-1/2-inch (40-mm) OD, close elbow to accommodate complete fixture.

2.11 STAINLESS STEEL, ELLIPTICAL WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of elliptical wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each stainless steel, elliptical wash fountain.

A. Wash Fountains <Insert drawing designation>: Stainless steel, elliptical receptor, one, two, or three wash station.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3401 option for elliptical wash fountain with one wash station. Retain model 3401ES for semicircular wash fountain with one wash station and contoured base.

Retain model 3402 for elliptical wash fountain with two wash stations. Retain model 3402ES for elliptical wash fountain with two wash stations and contoured base.

Retain model 3403 for elliptical wash fountain with three wash stations. Retain model 3403ES for elliptical wash fountain with three wash stations and contoured base.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware stainless steel, model [3401] [3401ES] [3402] [3402ES] [3403] [3403ES]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant fixtures are specified.

b. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.

b. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

c. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart.].
d. Number of Receptors: One.

e. Drain: Dome strainer.

5. Spray Head:

a. Material: Integral part of receptor back.

b. Number of Wash Stations: [One] [Two] [Three].

c. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASME A112.18.1/CSA B125.1

Verify option retained in "Control" Subparagraph below is available from listed manufacturers and for type of wash fountain required. Retain first option to specify manufacturer's standard product.

d. Control: [Individual push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering] actuation.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

e. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

f. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design. Soap dispensers are not available for off-floor-mounted models.

6. Soap Dispensers: Metering liquid soap pump for each wash station.

In "Pedestal Material and Finish" Subparagraph below, black vinyl-clad galvanized steel is manufacturer's standard for models 3401, 3402, and 3403 with optional stainless steel. Stainless steel is standard for models 3401ES, 3402ES, and 3403ES.

7. Pedestal Material and Finish: [Black vinyl-clad galvanized steel] [Stainless steel, No. 4 satin finish].

8. Supply Fittings: Wall.

9. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.

Accessories listed below are optional. Retain only subparagraphs needed to comply with Project design requirements.

10. Accessories:

a. Hose Bibb [left side] [right side]

11. Mounting: [Off-floor, on reinforced wall] [On-floor, with wall and floor mounting flanges on wash fountain base interior].
B. Wash Fountains <Insert drawing designation>: Stainless steel, elliptical receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model ELPS1 option for elliptical wash fountain with one wash station.

Retain model ELPS2 for elliptical wash fountain with two wash stations.
1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Wash-ware Ellipse model [ELPS1] [ELPS2]; <Insert product name or designation> or comparable product by one of the following:

a. Bradley Corporation.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:
a. ASME A112.19.3/CSA B45.4.

Retain first subparagraph below when ADA-compliant fixtures are specified.

b. ADA Standards for Accessible Design.

3. Electrical Components, Devices, and Accessories: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

In "Height to Rim" Subparagraph below, standard height is 34 inches (864 mm).

b. Height to Rim: 34 inches (864 mm) above floor.

In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

c. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart.].
d. Number of Receptors: One.

e. Faucet Punching: 4-inch (102-mm) centerset.

f. Drain Punching: 1-1/2-inch (40-mm) nominal diameter.

5. Mounting: Off-floor.

6. Enclosure:

a. Material: Type 304 stainless steel, secured with vandal-resistant fasteners, complete with removable access panel.

7. Valve Control:
Retain "Faucet" Subparagraph below when faucet to be provided by fixture manufacturer. Coordinate faucet style with applicable options in "Supply" Subparagraph below.
a. Faucet: [4-inch (102-mm) centerset with gooseneck spout and wrist blade handles] [4-inch (102-mm) centerset with gooseneck spout] [Sensor operated with J spout and deck plate] [Push-button metering with deck plate].
b. Supply: [Hot and cold] [Single temperature].

Retain "Sensor" Subparagraph below if retaining "sensor operated" option in "Faucet" Subparagraph above.

c. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Soap Dispensers" Subparagraph below when required for Project design.

8. Soap Dispensers:
a. Metering liquid soap pump for each wash station.

b. Sensor-operated for each wash station.

Accessories listed below are optional. Retain only subparagraphs needed to comply with Project design requirements.

9. Accessories:

a. Daisy strainer, tailpiece, plastic 1-1/2-inch (40-mm) P-trap.

b. Daisy strainer, tailpiece, chrome 1-1/2-inch (40-mm) P-trap.

2.12 STAINLESS STEEL, LINEAR WASH FOUNTAINS

Copy "Wash Fountains" Paragraph below and re-edit for each type of semicircular wash fountain required.

Insert letter and number combination to complete drawing designation. Use these designations on Drawings to identify each stainless steel, semicircular wash fountain.

A. Wash Fountains <Insert drawing designation>: Stainless steel, with linear, side-by-side receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model 3701 option for curved-front wash fountain with one wash station.

Retain model 3702 for curved-front wash fountain with two wash stations.

Retain model 3703 for curved-front wash fountain with three wash stations.

Retain model 3704 for curved-front wash fountain with four wash stations.

Retain model 3711 for straight-front wash fountain with one wash station.

Retain model 3712 for straight-front wash fountain with two wash stations.

Retain model 3713 for straight-front wash fountain with three wash stations.

Retain model 3714 for straight-front wash fountain with four wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International model [3701] [3702] [3703] [3704] [3711] [3712] [3713] [3714]; <Insert product name or designation> or comparable product by one of the following:

a. Metcraft Industries.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. ASMEA112.19.3/CSA B45.4.

Retain first subparagraph below for ADA Standards only when ADA-compliant fixtures are specified. Off-floor models are ADA compliant when mounted at proper height. On-floor models are not ADA compliant.

b. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

b. Material Type 304 stainless steel.

In "Height to Rim" Subparagraph below, standard height is 34 inches. Retain 31-inch "Junior" height when required by Project design and when ADA compliance is not required.

c. Height to Rim: [31 inches (787 mm)] [34 inches (864 mm)] above floor.

In "Finish" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

d. Finish: [No. 4 satin finish on exposed surfaces] [Powder coat; select color from manufacturer's standard color chart].

e. Number of Bowls: [One] [Two] [Three] [Four].
Retain first option in "Bowl Style, Curved-Front Wash Fountains" Subparagraph below for model 3701 to 3704 series standard construction. Retain one of other options to conform to Project design requirements.
f. Bowl Style, Curved-Front Wash Fountains: [Factory installed round, self-rimming basin] [Factory installed D-shape, self-rimming basin] [Welded, integral round bowl].

Retain first option in "Bowl Style, Straight-Front Wash Fountains" Subparagraph below for model 3711 to 3704 series standard construction. Retain one of other options to conform to Project design requirements.
g. Bowl Style, Straight-Front Wash Fountains: [Welded, integral round bowl] [Welded, integral trapezoidal bowl] [Welded, integral D-shaped bowl].

h. Drain: Grid strainer with 1-1/2-inch (40-mm) OD close elbow and tailpiece.

5. Faucets and Controls:

a. Material: Stainless steel or integral part of receptor back.

b. Number of Wash Stations: [One] [Two] [Three] [Four].

Retain one of first two subparagraphs below to require a field-mounted manual faucet or a factory-mounted push-button or sensor.

c. Field-Mounted Faucet Control: [Punched for field-mounted faucet with required number of holes and spacing] [4-inch (102-mm) centerset faucet with gooseneck spout and wrist blade handles].

d. Factory-Mounted Spout Control: 

1) Spout: [Deck-mounted "J" spout] [Deck-mounted "L" spout].

2) Control: [Individual, backsplash-mounted, metering push-button] [Individual, deck-mounted, metering push-button] [Individual, hardwired sensor] [Individual, battery-powered sensor] [Programmable piezo push-button, with electronic metering].

Retain "Sensor" Subparagraph below if retaining "Individual, hardwired sensor" or "Individual, battery-powered sensor" option in "Control" Subparagraph above.

e. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

Retain "Spray Nozzles" Subparagraph below when retaining "Faucet" or "Spout” above as provided by fixture manufacturer.

f. Spray Nozzles: At each wash station, provide 0.5 gpm (1.9 L/min.), vandal-resistant nozzle complying with ASMEA112.18.1/CSA B125.1.

In "Valve and Supply" Subparagraph below, select first option for standard configuration. Select second option for optional configuration.

g. Valve and Supply: [Hot and cold, ASSE 1070-compliant temperature and pressure balancing mixing valve, with integral checks and strainer] [Single-temperature with check stop and strainer].
6. Housing:

a. Material: Type 304 stainless steel.
b. Finish: [Stainless steel, No. 4 satin finish] [Black, vinyl-clad galvanized steel].
In "Pedestal" Subparagraph below, the standard pedestal material for off-floor models is stainless steel. Standard pedestal finish for on-floor model 3701 to 3704 series is No. 4 satin. Standard pedestal finish for on-floor model 3711 to 3714 series is black vinyl-clad, with the option for stainless steel.
7. Pedestal: [Off-floor stainless steel] [On-floor stainless steel] [On-floor with black, vinyl-clad steel enclosure].

a. Material: [Type 304 stainless steel] [Galvanized steel].
b. Finish: [Stainless steel, No. 4 satin finish] [Black, vinyl-clad galvanized steel].

8. Accessories:

a. Enclosed bottom.

b. Extended countertop: [Left hand] [Right hand] <Insert length>.
c. Mirror finish basin.

d. Basin overflow.

2-1/2-inch (64-mm) stainless steel narrow apron is available for off-floor models only.

e. 2-1/2-inch (64-mm) stainless steel narrow apron.

Scalloped front enclosure is available for model 3701 to 3704 series, off-floor fixtures only.

f. Scalloped front enclosure style.

Sloped front enclosure is available for model 3711 to 3714 series, off-floor fixtures only.

g. Sloped front enclosure style.

Marine edge is available for model 3711 to 3714 series fixtures only.

h. Marine edge.

Retain "Soap Dispensers" Subparagraph below when required for Project design.

9. Soap Dispensers: Metering liquid soap pump for each wash station.

10. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Drain Piping: 1-1/2-inch (40-mm) OD tailpiece.

11. Supply Fittings:

a. Supply Piping: From [bottom] [top] [wall].

12. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap and Drain Piping: NPS 1-1/2 (DN 40), P-trap to accommodate complete fixture.

In "Waste Outlet" Subparagraph below, retain first option for fixtures with one to three wash stations. Retain second option for fixtures with four wash stations.

c. Waste Outlet: [One wall waste connection] [Two wall waste connections].

B. Wash Fountains <Insert drawing designation>: Stainless steel, with linear, side-by-side receptor.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

Retain Acorn Engineering model DSW130 option for wash fountain trough with one wash station.

Retain model DSW260 for wash fountain trough with two wash stations.

Retain model DSW390 for wash fountain trough with three wash stations.

Retain model DSW4120 for wash fountain trough with four wash stations.

Retain model WSW130 for wash fountain trough with one wash station.

Retain model WSW260 for wash fountain trough with two wash stations.

Retain model WSW390 for wash fountain trough with three wash stations.

Retain model WSW4120 for wash fountain trough with four wash stations.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International Hand Wash Trough model [DSW130] [DSW260] [DSW390] [DSW4120] [WSW130] [WSW260] [WSW390] [WSW4120]; <Insert product name or designation> or comparable product by one of the following:

a. Metcraft Industries.

b. Willoughby Industries.

c. <Insert manufacturer's name>.

2. Standard:

a. ASME A112.19.3/CSA B45.4.
Retain first subparagraph below for ADA Standards only when ADA-compliant fixtures are specified.
b. ADA Standards for Accessible Design.

3. Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

4. Receptor:

a. Standard: ASME A112.19.3/CSA B45.4 for stainless steel receptor.

b. Material Type 304 stainless steel.
In "Height to Rim" Subparagraph below, standard height is 34 inches (864 mm).

c. Height to Rim: 34 inches (864 mm) above floor.
In "Finish or Color" Subparagraph below, retain first option for standard finish, exposed stainless steel. Retain second option for units with painted exposed surfaces.

d. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart].
e. Number of Receptors: One.
f. Faucet Punching: 4-inch (102-mm) centerset.
g. Drain Punching: 1-1/2-inch (40-mm) OD nominal diameter.

5. Mounting: Off-floor.

6. Enclosure:

a. Material: Type 304 stainless steel, secured with vandal-resistant fasteners.
b. Finish or Color: [No. 4 satin finish] [Powder coat; select color from manufacturer's standard color chart].
7. Valve Control:
Retain "Faucet" Subparagraph below when faucet to be provided by fixture manufacturer. Coordinate faucet style with applicable options in "Supply" Subparagraph below.

a. Faucet: [4-inch (102-mm) centerset with gooseneck spout and wrist blade handles] [4-inch (102-mm) centerset with gooseneck spout] [Sensor operated with J spout and deck plate] [Push-button metering with deck plate].
b. Supply: [Hot and cold] [Single-temperature].

Retain "Sensor" Subparagraph below if retaining "Sensor operated" option in "Faucet" Subparagraph above.

c. Sensor: ASME A112.18.1/CSA B125.1 and UL 1951.

8. Bottle Filler:
a. Quantity: [One] [Two] [Three] [Four].

b. Actuation: [Push-button] [Sensor-operated] [Sensor-operated with bottle counter display].
Retain "Soap Dispensers" Subparagraph below when required for Project design. Sensor-operated soap dispensers are not available for basis-of-design WSW wedge series trough.
9. Soap Dispensers: [Metering liquid soap pump] [sensor-operated] for each wash station.

Accessories listed below are optional. Retain only subparagraphs needed to comply with Project design requirements.

10. Accessories:

a. Daisy strainer, tailpiece, plastic 1-1/2-inch (40-mm) P-trap.

b. Daisy strainer, tailpiece, chrome 1-1/2-inch (40-mm) P-trap.

Retain subparagraph below for fixtures having two to four wash stations.

c. Splash guard.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in of water-supply, sanitary drainage, and vent piping systems to verify actual locations of piping connections before wash-fountain installation.

B. Examine walls and floors for suitable conditions where wash fountains will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Install wash fountains level and plumb in accordance with roughing-in drawings.

B. Set freestanding wash fountains on floor.

C. Install off-floor carrier supports, affixed to building substrate, for wall-mounted wash fountains.

D. Install accessible, wall-mounted wash fountains at mounting height for handicapped/elderly in accordance with ICC A117.1.

E. Install water-supply piping with shutoff valve on each supply to each wash fountain to be connected to domestic-water distribution piping. Use ball or gate valve. Install valves in locations where they can be easily reached for operation. Valves are specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Section 220523.15 "Gate Valves for Plumbing Piping."

F. Install trap and waste piping on each drain outlet of each wash fountain to be connected to sanitary drainage system.

G. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

H. Seal joints between fixtures and walls using sanitary-type, one-part, mildew-resistant, silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

3.3 CONNECTIONS

Coordinate piping installations and specialty arrangements with Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect wash fountains with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

B. Comply with requirements for water piping specified in Section 221116 "Domestic Water Piping."

C. Comply with requirements for soil and waste drainage piping[ and vent piping] specified in Section 221316 "Sanitary Waste and Vent Piping."

D. Install protective-shielding pipe covers and enclosures on exposed supplies and waste piping of accessible wash fountains. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

3.4 ADJUSTING

A. Operate and adjust wash fountains and controls. Replace damaged and malfunctioning wash fountains, fittings, and controls.

B. Adjust water pressure at faucets to produce proper flow.

C. Install fresh batteries in battery-powered, electronic-sensor mechanisms.

3.5 CLEANING AND PROTECTION

A. After installing wash fountains, inspect and repair damaged finishes.

B. Clean wash fountains, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

C. Provide protective covering for installed wash fountains and fittings.

D. Do not allow use of wash fountains for temporary facilities unless approved in writing by Owner.

END OF SECTION 224233
	WASH FOUNTAINS
	224233 - 38



