Copyright 2017 AIA
MasterSpec Full Length
06/17 (PM 06/21)
PRODUCT MASTERSPEC LICENSED BY DELTEK TO ACORN ENGINEERING; MORRIS GROUP INTERNATIONAL.

Copyright 2017 by The American Institute of Architects (AIA)

Exclusively published and distributed by DELTEK INC. for the AIA
This Product MasterSpec Section is licensed by DELTEK INC. to Acorn Engineering; Morris Group International ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Avitru or The American Institute of Architects (AIA). Neither AIA nor Avitru are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Acorn Engineering; Morris Group International, 15125 Proctor Ave., City of Industry, CA 91746; Phone: (626) 336-4561 | (800) 488-8999; Fax: <Insert Fax number>; Website: www.acorneng.com; Email: dyoung@acorneng.com.

For information about MasterSpec, contact Avitru at (800) 424-5080 or visit avitru.com

 HYPERLINK "http://" .

SECTION 224216.16 - COMMERCIAL SINKS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.

To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com.>
Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Service basins.

2. Service sinks.

3. Utility sinks.

4. Handwash sinks.

5. Sacristy sinks.

6. Sink faucets.

7. Laminar-flow, faucet-spout outlets.

8. Supports.

9. Supply fittings.

10. Waste fittings.

B. Related Requirements:

Retain subparagraph below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

1. Section 224100 "Residential Plumbing Fixtures" for residential sinks.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

1. Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for sinks.

2. Include rated capacities, operating characteristics[, electrical characteristics,] and furnished specialties and accessories.

B. Sustainable Design Submittals:

"Product Data" Subparagraph below applies to LEED (including v4), IgCC, ASHRAE 189.1, and Green Globes for fixtures and equipment that consume water.

1. Product Data: For water consumption.
1.3 INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below if coordination is required for installation of products and materials by separate installers.

A. Coordination Drawings: Counter cutout templates for mounting of counter-mounted lavatories.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance Data: For sinks to include in maintenance manuals.

1.5 MAINTENANCE MATERIAL SUBMITTALS

A. Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

1. Faucet Washers and O-Rings: Equal to [10] <Insert number> percent of amount of each type and size installed.

2. Faucet Cartridges and O-Rings: Equal to [5] <Insert number> percent of amount of each type and size installed.

PART 2 -  PRODUCTS

Manufacturers and products listed in SpecAgent and MasterWorks Paragraph Builder are neither recommended nor endorsed by the AIA or Avitru. Before inserting names, verify that manufacturers and products listed there comply with requirements retained or revised in descriptions and are both available and suitable for the intended applications. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

2.1 SERVICE BASINS

Caution: Service basins in this article require a drainage piping trap under the fixture. The trap is not part of the fixture fittings.

Copy "Service Basins" Paragraph below and re-edit for each type of terrazzo, floor-mounted service basin required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each service basin.

A. Service Basins <Insert drawing designation>: Terrazzo, floor mounted.

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to require a specific product or a comparable product from manufacturers listed.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Acorn Engineering; Morris Group International, Terrazzo-Ware Mop Sink model  [TCR-28] [TDF-24] [TDF-32] [TDF-36] [TDF-3624] [TNC-24] [TNC-32] [TNC-36] [TRH-242406] [TRH-242410] [TRH-362410] [TRH-363206] [TRH-303006] [TRH-323206] [TRH-362406] [TRH-363606] [TRH-363606-NC] [TSH-24] [TSH-32] [TSH-36] [TSH-3624]; <Insert product name or designation> or comparable product by one of the following:

a. Crane Plumbing, L.L.C.

b. Florestone Products Co., Inc.

c. Stern-Williams Co., Inc.
d. <Insert manufacturer's name>.

2. Fixture:

a. Standard:

1) IAPMO PS 99.

2) CSA B45.8/IAPMO Z403

In "Shape" and "Nominal Size" Subparagraphs below, retain shape and size options from the first five subparagraphs that follow. When multiple shapes or sizes are required by the project design, delete these subparagraphs and indicate fixture shapes and sizes in the Drawing fixture schedule.

b. Shape: [Square] [Rectangular] [Corner, with angled front] [Corner, with rounded, front] [Dropped front] <Insert shape>.

In "Nominal Size" Subparagraph below, retain fixture dimension options that correspond with fixture shape.

c. Nominal Size:

In ""Square" Subparagraph below, 30 by 30 inch (762 by 762 mm) fixture is available in 6-inch (150-mm) height only.

1) Square: [24 by 24 inches (610 by 610 mm)] [30 by 30 inches (762 by 762 mm)] [32 by 32 inches (813 by 813 mm)] [36 by 36 inches (915 by 915 mm)] <Insert dimensions>.

2) Rectangular: [36 by 24 inches (915 by 610 mm)] [36 by 32 inches (915 by 813 mm)] <Insert dimensions>.

3) Corner, with Angled Front: [24 by 24 inches (610 by 610 mm)] [32 by 32 inches (813 by 813 mm)] [36 by 36 inches (915 by 915 mm)] <Insert dimensions>.

4) Corner, with Rounded Front: [28 by 28 inches (711 by 711 mm)] <Insert dimensions>.

5) Dropped Front: [24 by 24 inches (610 by 610 mm)] [32 by 32 inches (813 by 813 mm)] [36 by 24 inches (915 by 610 mm)] [36 by 36 inches (915 by 915 mm)] <Insert dimensions>.

In "Height" Subparagraph below, retain fixture height values that correspond with manufacturer's offerings for fixtures specified.
d. Height: [6 inches (150 mm)] [10 inches (255 mm)] [12 inches (305 mm)] [12 inches (305 mm) with dropped front] <Insert dimensions>.

Retain "Tiling Flange" Subparagraph below, if required. Retain options for side(s) if basin is rectangular.

e. Tiling Flange, Integral, Stainless Steel:
1) Square or Rectangular Fixture: [Rear] [Left and rear] [Right and rear] [Three sides].

2) Corner Fixture: [Left] [Right] [Left and right].

Retain "Rim Guard" Subparagraph to suit project.

f. Rim Guard, Integral Stainless Steel: Top surface, each side.

g. Color: From manufacturer's standard color selection.

h. Drain: [Grid with NPS 3 (DN 80)] [Beehive dome strainer with NPS 3 (DN 80)] [Lint basket strainer with NPS 3 (DN 80)] outlet.

In "Accessories" Subparagraph below, retain only the subparagraphs needed to comply with project design requirements. Coordinate designated product submittal with accessory listed. When required, select only one of the selections for Rubber Drain Gasket, and for Bumper Guard.
3. Accessories: 

a. Rubber drain gasket for NPS 3 (DN 80) waste pipe.

b. Rubber drain gasket reducer for NPS 2 (DN 50) waste pipe.

c. Wall Bumper Guard: [Stainless steel] [Vinyl] wall bumper guard for <insert fixture size> fixture, [left] [right] [and] [rear] walls. Ship loose for field installation.
d. Tiling Flange: Provide factory-installed for <insert fixture size> fixture, [left] [right] [and] [rear] walls.
e. Mop hanger.
f. Hose: 36-inches (915-mm) long, with wall hanger.
A faucet with integral stops in shanks, vacuum breaker, hose-thread outlet, wall brace, and pail hook is recommended.

4. Faucet: [Furnished separately] [Chrome-plated utility faucet with manual hot water and cold water handles. Ship loose for field installation].
5. Mounting: On floor and flush to wall.

Copy "Service Basins" Paragraph below and re-edit for each type of plastic, floor-mounted service basin required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each service basin.

B. Service Basins <Insert drawing designation>: Plastic, floor mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by the following:

a. Zurn Industries, LLC.

b. <Insert manufacturer's name>.

2. Fixture:

a. Standard: IAPMO/ANSI Z124.6.

b. Material: [Cast polymer] <Insert material>.

c. Nominal Size: [24 by 36 by 10 inches (610 by 915 mm by 255 mm)] [36 by 36 by 10 inches (915 by 915 by 255 mm)] <Insert dimensions>.

In "Tiling Flange" Subparagraph below, verify availability if required and indicate side(s) if basin is rectangular.

d. Tiling Flange: [Not required] [On one side] [On two sides] [On three sides].

Indicate front dimension on rectangular basins if rim guard is to be only on one side.

e. Rim Guard: On [front] [all] top surfaces.

f. Color: [Not applicable] <Insert color>.

g. Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

3. Mounting: On floor and flush to wall.

A faucet with integral stops in shanks, vacuum breaker, hose-thread outlet, wall brace, and pail hook is recommended.

4. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

2.2 SERVICE SINKS

Copy "Service Sinks" Paragraph below and re-edit for each type of enameled, cast-iron, trap-standard-mounted service sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each service sink.

A. Service Sinks <Insert drawing designation>: Enameled, cast iron, trap standard mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. American Standard.

b. Commercial Enameling Company.

c. Gerber Plumbing Fixtures LLC.

d. Kohler Co.

e. Zurn Industries, LLC.

f. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.1/CSA B45.2.

b. Type: Service sink with back.

c. Back: [Two faucet holes] [Plain].

d. Nominal Size: [22 by 18 inches (560 by 457 mm)] [24 by 20 inches (610 by 508 mm)].

e. Color: White.

f. Mounting: [NPS 2 (DN 50)] [NPS 3 (DN 80)] P-trap standard with grid strainer inlet, cleanout, and floor flange.

g. Rim Guard: On front and sides.

A faucet with integral stops in shanks, vacuum breaker, hose-thread outlet, and pail hook is recommended. If retaining "Plain" option in "Back" Subparagraph above, specify a wall-mounted faucet and include a wall brace.

3. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

Coordinate "Support" Subparagraph below with "Supports" Article.

4. Support: [Type II sink carrier.] <Insert carrier>.

5. Lavatory Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Copy "Service Sinks" Paragraph below and re-edit for each type of vitreous-china, trap-standard-mounted service sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each service sink.

B. Service Sinks <Insert drawing designation>: Vitreous china, trap standard mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Crane Plumbing, L.L.C.

b. Kohler Co.

c. Zurn Industries, LLC.

d. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.2/CSA B45.1.

b. Type: Service sink with back.

c. Back: [Two faucet holes] [Plain].

d. Nominal Size: [22 by 20 inches (560 by 508 mm)] <Insert dimensions>.

e. Color: White.

f. Mounting: [NPS 2 (DN 50)] [NPS 3 (DN 80)] P-trap standard with grid strainer inlet, cleanout, and floor flange.

g. Rim Guard: On front and sides.

A faucet with integral stops in shanks, vacuum breaker, hose-thread outlet, and pail hook is recommended. If retaining "Plain" option in "Back" Subparagraph above, specify a wall-mounted faucet and include a wall brace.

3. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

Coordinate "Support" Subparagraph below with "Supports" Article.

4. Support: [Type II sink carrier.] <Insert carrier>.

5. Lavatory Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

Caution: Sinks in "Service Sinks" Paragraph below require a drainage piping trap under the fixture. The trap is not part of the fixture fittings.

Copy "Service Sinks" Paragraph below and re-edit for each type of enameled, cast-iron, floor-mounted service sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each service sink.

C. Service Sinks <Insert drawing designation>: Enameled, cast iron, floor mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by the following:

a. Zurn Industries, LLC.
b. <Insert manufacturer's name>.
2. Fixture:

a. Standard: ASME A112.19.1/CSA B45.2.

b. Style: With front apron and raised back.

c. Nominal Size: 28 by 28 inches (710 by 710 mm).

d. Color: White.

e. Drain: Grid with [NPS 2 (DN 50)] [NPS 3 (DN 80)] outlet.

f. Rim Guard: Coated wire.

A chrome-plated, solid-brass faucet with integral stops in shanks, vacuum breaker, hose-thread outlet, wall brace, and pail hook is recommended.

3. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

2.3 UTILITY SINKS

Caution: Sinks in this article are unsuitable for food preparation or for use as scullery sinks.

Copy "Utility Sinks" Paragraph below and re-edit for each type of stainless steel, counter-mounted utility sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each utility sink.

A. Utility Sinks <Insert drawing designation>: Stainless steel, counter mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advance Tabco.

b. Eagle Group.

c. Elkay Manufacturing Co.

d. Griffin Products, Inc.

e. Just Manufacturing.

f. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.3/CSA B45.4.

b. Type: Ledge back.

c. Number of Compartments: [One] [Two] [Three].

d. Overall Dimensions: <Insert dimensions>.

e. Metal Thickness: [0.050 inch (1.3 mm)] <Insert dimension>.

Retain "Compartment" Subparagraph below for single-compartment sinks.

f. Compartment:

1) Dimensions: <Insert dimensions>.

2) Drain: [Grid with NPS 1-1/2 (DN 40) tailpiece and twist drain] [Grid with NPS 2 (DN 50) tailpiece and twist drain] [NPS 1-1/2 (DN 40) tailpiece with stopper] <Insert drain>.

3) Drain Location: [Centered in compartment] [Near back of compartment] [Near left side of compartment] [Near right side of compartment] <Insert location>.

Retain "Each Compartment" Subparagraph below for multiple-compartment sinks.

g. Each Compartment:

1) Dimensions: <Insert dimensions>.

2) Drains: [Grid with NPS 1-1/2 (DN 40) tailpiece and twist drain] [Grid with NPS 2 (DN 50) tailpiece and twist drain] [NPS 1-1/2 (DN 40) tailpiece with stopper] <Insert drain>.

3) Drain Location: [Centered in compartment] [Near back of compartment] <Insert location>.

3. Faucet(s): <Insert sink-faucet designation from "Sink Faucets" Article>.

a. Number Required: [One] [Two].

b. Mounting: On ledge.

4. Supply Fittings:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Supplies: Chrome-plated brass compression stop with inlet connection matching water-supply piping type and size.

1) Operation: [Loose key] [Wheel handle] <Insert type>.

2) Risers: NPS 1/2 (DN 15), [chrome-plated, rigid-copper pipe] [chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided or corrugated stainless steel flexible hose].

5. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap(s):

1) Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

Retain one of two "Material" subparagraphs below. First subparagraph is for NPS 1-1/2 or NPS 2 (DN 40 or DN 50) traps.

2) Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] <Insert trap type>; and chrome-plated brass or steel wall flange.

Retain "Material" Subparagraph below only for NPS 1-1/2 (DN 40) traps.

3) Material: Stainless steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless steel tube to wall; and stainless steel wall flange.

If retaining "Continuous Waste" Subparagraph below, show arrangement on Drawings.

c. Continuous Waste:

1) Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

2) Material: Chrome-plated, 0.032-inch- (0.83-mm-) thick brass tube.

6. Mounting: On counter with sealant.

Copy "Utility Sinks" Paragraph below and re-edit for each type of stainless steel, freestanding utility sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each utility sink.

B. Utility Sinks <Insert drawing designation>: Stainless steel, freestanding.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advance Tabco.

b. AERO Manufacturing Company.

c. Amtekco Industries, Inc; a Wasserstrom Company.

d. Eagle Group.

e. Elkay Manufacturing Co.

f. Griffin Products, Inc.

g. Just Manufacturing.

h. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.3/CSA B45.4.

b. Type: With backsplash.

c. Number of Compartments: [One] [Two] [Three].

d. Overall Dimensions: <Insert dimensions>.

e. Metal Thickness: [0.050 inch (1.3 mm)] [0.063 inch (1.6 mm)] <Insert dimension>.

Retain "Compartment" Subparagraph below for single-compartment sinks.

f. Compartment:

1) Dimensions: <Insert dimensions>.

2) Drain: [Grid with NPS 1-1/2 (DN 40) tailpiece and twist drain] [Grid with NPS 2 (DN 50) tailpiece and twist drain] [NPS 1-1/2 (DN 40) tailpiece with stopper] <Insert drain>.

3) Drain Location: [Centered in compartment] [Near back of compartment] [Near left side of compartment] [Near right side of compartment] <Insert location>.

Retain "Each Compartment" Subparagraph below for multiple-compartment sinks.

g. Each Compartment:

1) Dimensions: <Insert dimensions>.

2) Drains: [Grid with NPS 1-1/2 (DN 40) tailpiece and twist drain] [Grid with NPS 2 (DN 50) tailpiece and twist drain] [NPS 1-1/2 (DN 40) tailpiece with stopper] <Insert drain>.

3) Drain Location: [Centered in compartment] [Near back of compartment] <Insert location>.

h. Drainboard(s): [Not required] [Both] [Left] [Right] side(s).

1) Dimensions Each: [Not applicable] <Insert dimensions>.

3. Supports: Adjustable-length steel legs.

4. Faucet(s): <Insert sink-faucet designation from "Sink Faucets" Article>.

a. Number Required: [One] [Two].

b. Mounting: On backsplash.

5. Supply Fittings:

a. Standard: ASME A112.18.1/CSA B125.1.

b. Supplies: Chrome-plated brass compression stop with inlet connection matching water-supply piping type and size.

1) Operation: [Loose key] [Wheel handle] <Insert type>.

2) Risers: NPS 1/2 (DN 15), [chrome-plated, rigid-copper pipe] [chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided or corrugated stainless steel flexible hose].

6. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. Trap(s):

1) Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

Retain one of two "Material" subparagraphs below. First subparagraph is for NPS 1-1/2 or NPS 2 (DN 40 or DN 50) traps.

2) Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] <Insert trap type>; and chrome-plated brass or steel wall flange.

Retain "Material" Subparagraph below only for NPS 1-1/2 (DN 40) traps.

3) Material: Stainless steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless steel tube to wall; and stainless steel wall flange.

If retaining "Continuous Waste" Subparagraph below, show arrangement on Drawings.

c. Continuous Waste:

1) Size: [NPS 1-1/2 (DN 40)] [NPS 2 (DN 50)].

2) Material: Chrome-plated, 0.032-inch- (0.83-mm-) thick brass tube.

2.4 HANDWASH SINKS

Retain this article only if handwash sinks are not specified in Section 114000 "Foodservice Equipment."

Copy "Handwash Sinks" Paragraph below and re-edit for each type of stainless steel, wall-mounted handwash sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each handwash sink.

A. Handwash Sinks <Insert drawing designation>: Stainless steel, wall mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Advance Tabco.

b. AERO Manufacturing Company.

c. Amtekco Industries, Inc; a Wasserstrom Company.

d. Eagle Group.

e. Elkay Manufacturing Co.

f. Griffin Products, Inc.

g. Just Manufacturing.

h. Sloan Valve Company.

i. <Insert manufacturer's name>.

2. Fixture:

a. Standards: ASME A112.19.3/CSA B45.4 and NSF/ANSI 2.

b. Type: Basin with radius corners, back for faucet, and support brackets.

c. Nominal Size: 17 by 16 by 5 inches (432 by 406 by 127 mm).

3. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

4. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

5. Waste Fittings: Comply with requirements in "Waste Fittings" Article.

Coordinate "Support" Subparagraph below with "Supports" Article.

6. Support: [Type II sink carrier.]<Insert carrier>.

7. Lavatory Mounting Height: [Standard] [Child] [Handicapped/elderly in accordance with ICC A117.1].

2.5 SACRISTY SINKS

Sacristy sinks are custom-made products. Fixture specified below is typical, but one-compartment sinks and faucets can be used.

Copy "Sacristy Sinks" Paragraph below and re-edit for each type of stainless steel, two-bowl, counter-mounted sacristy sink required.

Insert number to complete drawing designation. Use these designations on Drawings to identify each sacristy sink.

A. Sacristy Sinks <Insert drawing designation>: Stainless steel, two bowl, counter mounted.

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Elkay Manufacturing Co.

b. Just Manufacturing.

c. <Insert manufacturer's name>.

2. Fixture:

a. Standard: ASME A112.19.3/CSA B45.4.

b. Type: Self-rimming.

c. Material: Stainless steel.

d. Nominal Size: Approximately [22 by 42 inches (560 by 1070 mm)] <Insert dimensions>.

e. Cover: Hinged with lock on [left] [right] bowl.

3. Faucet: <Insert sink-faucet designation from "Sink Faucets" Article>.

a. Location: On [left] [right] bowl.

4. Supply Fittings: Comply with requirements in "Supply Fittings" Article.

5. Waste Fittings:

a. Standard: ASME A112.18.2/CSA B125.2.

b. [Left] [Right] Bowl:

1) Drain: Cup with stopper and NPS 1-1/2 (DN 40) tailpiece.

2) Drain Piping: NPS 1-1/2 (DN 40) chrome-plated, tubular-brass direct waste without trap, separate waste piping, and wall flange.

c. Opposite Bowl:

1) Drain: Grid with stopper and NPS 1-1/2 (DN 40) tailpiece.

2) Trap:

a) Size: NPS 1-1/2 (DN 40).

b) Material: Comply with requirements in "Waste Fittings" Article.

2.6 SINK FAUCETS

A. NSF Standard: Comply with NSF 372 for faucet-spout materials that will be in contact with potable water.

Copy "Sink Faucets" Paragraph below and re-edit for each type of manual-type, single-control and two-lever-handle mixing valve sink faucet required.

Insert faucet designations. Use these designations in sinks articles to identify each sink faucet.

B. Sink Faucets: Manual type, [single control] [two lever handle] <Insert type> mixing valve.

1. Commercial, Solid-Brass Faucets <Insert faucet designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1) American Standard.

2) Bradley Corporation.

3) Chicago Faucets; Geberit Company.

4) Delta Faucet Company.

5) Elkay Manufacturing Co.

6) GROHE America, Inc.

7) Just Manufacturing.

8) Kohler Co.

9) Moen Incorporated.

10) Sloan Valve Company.

11) Speakman Company.

12) T&S Brass and Bronze Works, Inc.

13) Zurn Industries, LLC.

14) <Insert manufacturer's name>.

2. General-Duty, Solid-Brass Faucets <Insert faucet designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1) American Standard.

2) Bradley Corporation.

3) BrassTech Inc.

4) Central Brass Company.

5) Chicago Faucets; Geberit Company.

6) Danze, Inc.

7) Delta Faucet Company.

8) Eljer, Inc.

9) Elkay Manufacturing Co.

10) Franke Consumer Products, Inc.

11) Gerber Plumbing Fixtures LLC.

12) Griffin Products, Inc.

13) GROHE America, Inc.

14) Hansgrohe USA.

15) Hydrotek International, Inc.

16) Intersan Manufacturing Company.

17) Just Manufacturing.

18) Kohler Co.

19) Matco-Norca.

20) Moen Incorporated.

21) Price Pfister, Inc.

22) Speakman Company.

23) T&S Brass and Bronze Works, Inc.

24) WhiteRock Corp.

25) Wolverine Brass, Inc.

26) Zurn Industries, LLC.

27) <Insert manufacturer's name>.

3. Copper- or Brass-Underbody Faucets <Insert faucet designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1) American Standard.

2) Briggs Plumbing Products, Inc.

3) Danze, Inc.

4) Delta Faucet Company.

5) Eljer, Inc.

6) FNW; Ferguson Enterprises, Inc.

7) Gerber Plumbing Fixtures LLC.

8) Matco-Norca.

9) Moen Incorporated.

10) Sterling.

11) WhiteRock Corp.

12) Wolverine Brass, Inc.

13) <Insert manufacturer's name>.

4. Standard: ASME A112.18.1/CSA B125.1.

5. General: Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture hole punchings; coordinate outlet with spout and sink receptor.

Option in "Body Type" Subparagraph below must match fixture hole punching.

6. Body Type: [Centerset] [Widespread] [Single hole] <Insert type>.

7. Body Material: [Commercial, solid brass] [General-duty, solid brass] [Copper or brass underbody].

8. Finish: [Chrome plated] [Polished chrome plate] <Insert finish>.

First option in "Maximum Flow Rate" Subparagraph below generally applies to sinks. Second option applies to service sinks.

9. Maximum Flow Rate: [2.2 gpm (8.3 L/min.)] [4.0 gpm (15 L/min.)] <Insert value>.

10. Handle(s): [Lever] [Cross, four arm] [Wrist blade, 4 inches (102 mm)] [Elbow, 6 inches (152 mm)] [Not applicable] <Insert handles>.

11. Mounting Type: [Deck, concealed] [Deck, exposed] [Back/wall, exposed].

12. Spout Type: [Rigid, solid brass] [Rigid, solid brass with wall brace] [Swing, round tubular] [Swing, shaped tube] [Swing, solid brass] [Rigid gooseneck] [Swivel gooseneck] <Insert type>.

Retain first option in "Vacuum Breaker" Subparagraph below for service-sink and other faucets with hose-thread outlet.

13. Vacuum Breaker: [Required] [Not required] for hose outlet.

14. Spout Outlet: [Aerator] [Laminar flow] [Hose thread in accordance with ASME B1.20.7] [Plain end] [Spray] <Insert type>.

Copy "Sink Faucets" Paragraphs below and re-edit for each automatic, sensor-operated, mixing valve sink faucet required.

Insert faucet designations. Use these designations in sinks articles to identify each sink faucet.

C. Sink Faucets: Automatic, sensor-operated type, [120 V ac hard-wired] [battery-powered] <Insert type>, mixing valve.

1. Commercial, Solid-Brass Faucets <Insert faucet designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1) American Standard.

2) Bradley Corporation.

3) Chicago Faucets; Geberit Company.

4) Delta Faucet Company.

5) Elkay Manufacturing Co.

6) GROHE America, Inc.

7) Just Manufacturing.

8) Kohler Co.

9) Moen Incorporated.

10) Sloan Valve Company.

11) Speakman Company.

12) T&S Brass and Bronze Works, Inc.

13) Zurn Industries, LLC.

14) <Insert manufacturer's name>.

2. General-Duty, Solid-Brass Faucets <Insert faucet designation>:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

a. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1) American Standard.

2) Bradley Corporation.

3) Chicago Faucets; Geberit Company.

4) Delta Faucet Company.

5) Hydrotek International, Inc.

6) Intersan Manufacturing Company.

7) Just Manufacturing.

8) Kohler Co.

9) Moen Incorporated.

10) Speakman Company.

11) T&S Brass and Bronze Works, Inc.

12) Wolverine Brass, Inc.

13) Zurn Industries, LLC.

14) <Insert manufacturer's name>.

3. Standard: ASME A112.18.1/CSA B125.1.

4. General: Coordinate faucet inlets with supplies and fixture hole punchings; coordinate outlet with spout and sink receptor.

Option in "Body Type" Subparagraph below must match fixture hole punching.

5. Body Type: [Centerset] [Single hole] <Insert type>.

6. Body Material: [Commercial, solid brass] [General-duty, solid brass].

7. Finish: [Chrome plated] [Polished chrome plate] <Insert finish>.

8. Maximum Flow Rate: [0.5 gpm (1.9 L/min.)] <Insert value>.

9. Mounting Type: [Deck] [Back/wall].

10. Spout Type: [Rigid spout] [Rigid gooseneck] <Insert type>.

11. Spout Outlet: [Aerator] [Laminar flow] <Insert type>.

12. Thermostatic Mixing Valve: [Above deck,] [Below deck,] [single temperature] [adjustable temperature manual side handle, with hot/cold water indicators], with check valves.

13. Control module: [Above deck,] [Below deck,] water-resistant module with internal flow setting switches.

2.7 LAMINAR-FLOW, FAUCET-SPOUT OUTLETS

Spout outlets in this article are non-aerating, produce less splashing than conventional faucet spouts, and help reduce bacteria growth.

A. NSF Standard: Comply with NSF 372 for faucet-spout-outlet materials that will be in contact with potable water.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

B. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

1. AM Conservation Group, Inc.

2. Chronomite Laboratories, Inc; a division of Morris Group International.

3. NEOPERL, Inc.

4. T&S Brass and Bronze Works, Inc.

5. <Insert manufacturer's name>.

C. Description: Chrome-plated brass, faucet-spout outlet that produces non-aerating, laminar stream. Include external or internal thread that mates with faucet outlet for attachment to faucets where indicated and flow-rate range that includes flow of faucet.

2.8 SUPPORTS

A. Type II Sink Carrier:

Retain "Manufacturers" Subparagraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

1. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

a. Jay R. Smith Mfg Co; a division of Morris Group International.

b. Josam Company.

c. MIFAB, Inc.

d. Wade Drains.

e. WATTS.

f. Zurn Industries, LLC.

g. <Insert manufacturer's name>.

2. Standard: ASME A112.6.1M.

2.9 SUPPLY FITTINGS

A. NSF Standard: Comply with NSF 372 for supply-fitting materials that will be in contact with potable water.

B. Standard: ASME A112.18.1/CSA B125.1.

C. Supply Piping: Chrome-plated brass pipe or chrome-plated copper tube matching water-supply piping size. Include chrome-plated brass or stainless steel wall flange.

D. Supply Stops: Chrome-plated brass, one-quarter-turn, ball-type or compression valve with inlet connection matching supply piping.

Retain "Wheel handle" option in "Operation" Paragraph below for one-quarter-turn, ball-type stops.

E. Operation: [Loose key] [Wheel handle] <Insert type>.

F. Risers:

1. [NPS 3/8 (DN 10)] [NPS 1/2 (DN 15)].

2. [Chrome-plated, rigid-copper pipe] [Chrome-plated, soft-copper flexible tube] [ASME A112.18.6, braided or corrugated stainless steel flexible hose].

2.10 WASTE FITTINGS

A. Standard: ASME A112.18.2/CSA B125.2.

B. Drain: Grid type with NPS 1-1/2 (DN 40) offset and straight tailpiece.

C. Trap:

1. Size: NPS 1-1/2 (DN 40).

Retain one of two "Material" subparagraphs below.

2. Material: Chrome-plated, [two-piece, cast-brass trap and swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [two-piece, cast-brass trap and ground-joint swivel elbow with 0.032-inch- (0.83-mm-) thick brass tube to wall] [one-piece, cast-brass trap with swivel 0.029-inch- (73-mm-) thick tubular brass wall bend] <Insert trap type>; and chrome-plated brass or steel wall flange.

3. Material: Stainless steel, two-piece trap and swivel elbow with 0.012-inch- (0.30-mm-) thick stainless steel tube to wall; and stainless steel wall flange.

2.11 GROUT

A. Standard: ASTM C1107/C1107M, Grade B, post-hardening and volume-adjusting, dry, hydraulic-cement grout.

B. Characteristics: Nonshrink; recommended for interior and exterior applications.

C. Design Mix: 5000-psi (34.5-MPa), 28-day compressive strength.

D. Packaging: Premixed and factory packaged.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in of water supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before sink installation.

B. Examine walls, floors, and counters for suitable conditions where sinks will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Install sinks level and plumb in accordance with roughing-in drawings.

B. Install supports, affixed to building substrate, for wall-hung sinks.

Indicate on Drawings those sinks that are required to be accessible.

C. Install accessible wall-mounted sinks at handicapped/elderly mounting height in accordance with ICC/ANSI A117.1.

D. Set floor-mounted sinks in leveling bed of cement grout.

E. Install water-supply piping with stop on each supply to each sink faucet.

1. Exception: Use ball or gate valves if supply stops are not specified with sink. Comply with valve requirements specified in Section 220523.12 "Ball Valves for Plumbing Piping" and Section 220523.15 "Gate Valves for Plumbing Piping."

2. Install stops in locations where they can be easily reached for operation.

F. Install wall flanges or escutcheons at piping wall penetrations in exposed, finished locations. Use deep-pattern escutcheons if required to conceal protruding fittings. Comply with escutcheon requirements specified in Section 220518 "Escutcheons for Plumbing Piping."

G. Seal joints between sinks and counters, floors, and walls using sanitary-type, one-part, mildew-resistant silicone sealant. Match sealant color to fixture color. Comply with sealant requirements specified in Section 079200 "Joint Sealants."

Indicate on Drawings those sinks that are required to be accessible.

H. Install protective shielding pipe covers and enclosures on exposed supplies and waste piping of accessible sinks. Comply with requirements in Section 220719 "Plumbing Piping Insulation."

3.3 CONNECTIONS

Coordinate piping installations and specialty arrangements with schematics on Drawings and with requirements specified in piping systems. If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Connect sinks with water supplies, stops, and risers, and with traps, soil, waste, and vent piping. Use size fittings required to match fixtures.

B. Comply with water piping requirements specified in Section 221116 "Domestic Water Piping."

C. Comply with soil and waste piping requirements specified in Section 221316 "Sanitary Waste and Vent Piping."

3.4 ADJUSTING

A. Operate and adjust sinks and controls. Replace damaged and malfunctioning sinks, fittings, and controls.

B. Adjust water pressure at faucets to produce proper flow.

3.5 CLEANING AND PROTECTION

A. After completing installation of sinks, inspect and repair damaged finishes.

B. Clean sinks, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.

C. Provide protective covering for installed sinks and fittings.

D. Do not allow use of sinks for temporary facilities unless approved in writing by Owner.

END OF SECTION 224216.16
	COMMERCIAL SINKS
	224216.16 - 1



